FUNCTIONAL SPECIFICATIONS

DOCUMENT

| Project Name | Network Customer Data Input for AFC Reservation Dispatch |  |  |
| --- | --- | --- | --- |
| Project Number |  | Version # | 2.0 |
| Date | August 22, 2008 |  |  |
| Author(s) | Entergy Technology Delivery |  |  |
| Project Lead |  |  |  |
| Customer | Entergy Transmission Network Customers |  |  |

DOCUMENT STORAGE

Copies of all documentation relative to this project will be stored, in electronic and printed, format in the Transmission Technology Delivery Department.

Introduction	4

Introduction	4

Fundamental Change	4

Network Customer Participation	4

Data Transfer Strategy	4

Clarifications	5

Network Customer Data File Requirements	5

Load Forecast Data File - Required	5

Data File Structure	5

Data Record Format	6

Reservation Dispatch Options	6

Default Reservation Dispatch Option	6

Default Process	6

TSR Stack Dispatch Option	7

Data File Structure	7

Data Record Format	8

TSR Hourly Dispatch Option	9

Data File Structure	9

Data Record Format	9

Unit Commitment Dispatch Option	10

Data File Structure	10

Data Record Format	10

General Requirements	11

Data File Transfer Process	12

Implementation Milestones	17

Appendix – Data File Examples	18

Overview	18

Load Forecast Data File Example	19

Data File Beginning with Record 1:	19

Unit Commitment Data File Example	21

Data File Beginning with Record 1:	21

TSR Stack Data File Example	24

Data File Beginning with Record 1:	24

TSR Hourly Data File Example	29

Data File Beginning with Record 1:	29

Table 1 LF File Field Formats	6

Table 2 Stack File Tag Formats	8

Table 3 Stack File Field Formats	8

Table 4 Hourly File Field Formats	9

Table 5 UC File Field Formats	11

Table 6 General Requirements	12

Table 7 Implementation Milestones	17

Table 8 Generation Unit Table	18

# Introduction

On September 19, 2006, Entergy Transmission received approval from the Near-Term Transmission Issues Working Group (NTTIWG) on a proposal to alter the dispatch methodology inherent to the Available Flowgate Capacity (AFC) process for the granting of Transmission service on the Entergy grid.  This proposal will utilize an alternate method to mathematically dispatch resources to match load projections for AFC models.

NTTIWG is sponsored and chaired by Entergy’s Independent Coordinator of Transmission (ICT).  Membership includes Entergy internal and external stakeholders involved in scheduling access to Entergy’s Transmission system.

## Fundamental Change

At the heart of the change in methodology are two fundamental principles.  First, the proposal approved in September seeks to balance resources and load by Network Customer instead of the previous method which seeks to solve by Control Area.  Second, the method to balance resources and load by Network Customer will be selected by the Network Customer.  This new strategy will be referred to as the “AFC Reservation Dispatch Methodology” in future references.

## Network Customer Participation

Significant software changes are required to implement the AFC Reservation Dispatch Methodology which will be jointly performed by Entergy Transmission and the AFC vendor, Areva.  In addition, the success of this methodology will be dependent upon the participation of all of Entergy Transmission’s Network Customers in the preparation of both load forecast projections and desired dispatch strategies.  This document describes the options available to the Network Customer as well as the data interface requirements and implementation tips and guidance.

## Data Transfer Strategy

The Network Customers are to provide data files, as described within, to Entergy Transmission for implementation of this reservation dispatch strategy.  A common file transfer mechanism will be provided to allow the Network Customers to “upload” files into the Entergy Transmission AFC pre-processor mechanism.  Once the individual data files are assembled and validated, the resulting data set will be used to initiate the AFC calculation process.  The original data provided by the Network Customers and the resulting assembled AFC pre-processor data sets will be considered private and will be secured from access by unauthorized companies and/or personal in accordance with FERC Standards of Conduct rules on information handling.  Only authorized Entergy Transmission staff and relevant contractors associated with the AFC process will have access to the original data files as well as to the assembled data sets.  Please note, the ICT is a recognized contractor to Entergy Transmission for the purpose of tariff administration.  Accordingly, the ICT will have access as warranted to any or all data related to the AFC process.

## Clarifications

This document refers to the methodology used by the AFC engine to balance resources with load and losses in order to solve power flow models for planning purposes and the assignment of AFC values to flowgates within the Entergy Transmission network model.  The term “dispatch” is used as the mathematical procedure to assign resources or portions of resources as directed by this process for the time points throughout the AFC operating and planning models.  The term “dispatch” is not meant to infer actual real-time generation dispatch for resources as is currently done under real-time operations.

The scope of this document is to provide enough information to allow Network Customers to prepare the proper data files and to describe the file upload mechanism to be used.  While some discussion of the dispatch process is unavoidable in such a document, detailed descriptions of how AFC will be changed to satisfy the updated methodology approved by the NTTWIG are out of scope.

# Network Customer Data File Requirements

The AFC Reservation Dispatch Methodology requires all Network Customers to provide load forecast data to Entergy by electronic means on a business daily basis, as specified within this document.  Optionally, Network Customers are given the opportunity to select from a variety of methods to direct how the AFC process would balance load and resources for that particular Network Customer.

This section describes the file format requirements for load forecast data and the available dispatch options.

## Load Forecast Data File - Required

Each business day, each Network Customer is required to provide a data file containing load forecasts for that Network Customer’s area over the next 35 days.  The data file shall contain hourly peak projected load including losses for the first 11 days followed by daily peak projected load including losses for the remaining 24 days to complete a 35 day report.

### Load Forecast Data File Structure

The data file shall contain one record per row.  Each record is to be fully contained within one row.  Each record received will be assumed to be of the format specified in the “Data Record Format” subsection below.

The data file shall contain one record per hour for the first 11 days beginning with the current day followed by one record per day for the remaining 24 days.  Thus, a data file will contain 264 (24 x 11) hourly records followed by 24 (1 x 24) daily records for a total of 288 records.

For the first 11 days, an hourly date and time stamp will be used for each hour to designate the MW value that is to represent the maximum load expected during the hour that starts with the first minute of the hour of the time given.  For the remaining 24 days, the time stamp provided is expected to be the hour which will contain the peak load for that day.

The data records shall be provided in ascending chronological order.

### Load Forecast Data Record Format

Each data record for Load Forecast data file shall contain the following fields beginning with the first column or character position of the record:

hour, nc name, MW

Comment lines are supported provided the “!” character is contained in the first column or character position of the record.  Comment lines are ignored by input processing.

The attributes for each record are defined in the table below:

| Field Name | Field Description | Format | Length | Units |
| --- | --- | --- | --- | --- |
| hour | Beginning hour date and time. The record will pertain to the first minute of the hour specified. | dd-mmm-yyyy-hh-mm | Fixed char(17) | Date &Time |
| nc name | Network Customer name as defined on Entergy’s OASIS site | Alpha-numeric | Variable char(16) | Identifier |
| MW | MW load including losses for the given hour. | Number | Variable int(5), e.g. 99999 | MW |

Table 1 LF File Field Formats

## Reservation Dispatch Options

Each business day, the Network Customer is required to provide sufficient resources for dispatch by the AFC Reservation Dispatch Methodology process to meet the load including losses provided in the Load Forecast Data File for that Network Customer.

The Network Customer may select from several options when providing resource information to the AFC process.  These options are introduced in the following subsections.  Please note that each option has a varying degree of complexity and detail required in the data files produced by the Network Customer.

## Default Reservation Dispatch Option

Apart from the Load Forecast data file, the default dispatch option does not require additional data files or action on the part of the Network Customer.  In the absence of a valid data file under the other dispatch options, the process will default to this option for dispatch by the AFC process.  This option is by far the simplest for the Network Customer to implement though this option limits the Network Customer’s opportunity to direct how resources are dispatched to meet load for planning models.

### Default Dispatch Process

A summarization of the default dispatch process is as follows:

- The Network Customer defines projected load according to the Load Forecast data file format.

- If a Unit Commitment file is provided and is valid, dispatch the units based on data in the Network Customer’s UC data file.

- If a short fall exists, search OASIS in reverse queue order (LIFO) for TSR’s that serve the Network Customer’s load and dispatch the TSR until the short fall is satisfied or the TSR is fully dispatched.

- For cases where network resources are designated in the UC file and in the TSR files, steps will be taken to ensure the proper dispatch.

- If a short fall continues to exist, continue to dispatch OASIS TSR’s for that Network Customer until the short fall is satisfied or the list of TSR’s is exhausted.

- If a short fall continues to exist, then the short fall is assigned to the balancing authority.

- If a short fall continues to exist, a power flow solution for that time point may not be able to solve.

## TSR Stack Dispatch Option

The TSR Stack Dispatch option allows the Network Customer to nominate and order specific TSR’s to be used to match the forecast load for the time point provided in the Load Forecast Data File.  For the first 11 days, there are two time spans for each day, “On Peak” and “Off Peak”.  For the following 24 days, only one time span (“Peak Hour”) is specified.

In this approach, TSR’s are “fully consumed” to satisfy the designated load requirement before processing continues to the next TSR in the Network Customer’s provided list.  TSR’s are processed in the order they are recorded in the Stack Dispatch data file.

The cumulative amount of the TSR capacity for each time block (i.e. “Off Peak”, “On Peak”, and “Peak Hour”) provided in the TSR Stack Dispatch file is expected to meet or exceed the forecast load requirement from the corresponding hour or day in the Load Forecast data file.  Should a short-fall exist, the Default Dispatch Process will be used to satisfy the remaining load requirement for that time point.

### Reservation Stack Data File Structure

The data file shall contain one record per row.  Each record is to be fully contained within one row.  Each record received will be assumed to be of the format specified in the “Data Record Format” subsection below.

This format follows the consistent strategy of defining the first 11 days, which correspond to hourly load projections, differently than the remaining 24 days, which correspond to the daily load projections, though the hourly section is simplified.  For the first 11 days, the records shall be ordered as date, OFF PEAK indicator, tsr  [,date, OFF PEAK indicator, tsr], date, ON PEAK indicator, tsr [, date, ON PEAK indicator, tsr].  For the remaining 24 days, the records shall be ordered as date, ON PEAK indicator, tsr [,date, ON PEAK indicator , tsr.  It is expected there will be multiple TSR records for many if not every time point thus increasing the overall file length accordingly.  See the Appendix for an example of this data file.

The data records shall be provided in ascending chronological order.

### Reservation Stack Data Record Format

Each data record for TSR Stack data file shall contain the following fields beginning with the first column or character position of the record:

dd-mon-yyyy, PEAK (1 or 0), TSR#

Comment lines are supported provided the “!” character is contained in the first column or character position of the record.  Comment lines are ignored by input processing.

The attributes for each record are defined in the table below:

| Tag Name | Tag Description | Format | Length | Units |
| --- | --- | --- | --- | --- |
| Date | Beginning hour date. The record will pertain to the first minute of the day specified. | dd-mmm-yyyy | Fixed char(13) | Date |
| Peak | Indicator that subsequent records pertain to the off peak or on peak hours for the given date. “Off Peak” hours are hours beginning 0 through 5, plus hours beginning 22 and 23 for a total of 8 hours each day. This is the first six hours and last two hours each day. “On Peak” hours are hours beginning 6 through 21 for a total of 16 hours each day. | 0 for off peak hours, 1 for on peak hours | Variable Int(1), e.g. 0 or 1 | identifier |
| tsr | Transmission Service Request number as defined on Entergy’s OASIS site. | Number | Variable int(8), e.g. 99999999 | Identifier |

Table 2 Stack File Field Formats

## TSR Hourly Dispatch Option

The TSR Hourly Dispatch option allows the Network Customer to nominate the specific TSR’s and the specific dispatch amount to be used for each TSR in order to meet the load requirement designated in the Load Forecast data file.  The Network Customer may provide a different set of TSR’s and a different dispatch amount for each TSR for every time point over the 35 day period of the Load Forecast data file.  Individual TSR’s are not required to be fully dispatched as is the case for the Stack Option.

In this approach, the TSR list for each hour or day is taken as a whole set meaning that TSR order within any time period is meaningless.  There are no provisions to account for scenarios when the TSR list cumulative dispatch for any time point does not match the forecast load requirement.

This option is incompatible with any other option.

The cumulative amount of the TSR dispatch MW provided in the TSR Hourly Dispatch file must meet the forecast load requirement from the corresponding time point in the Load Forecast data file.  Should this not be true for any hour or day, the TSR list for that time period will be rejected and the Default Dispatch Process will be used to satisfy the load requirement for that time point.

### Hourly Dispatch Data File Structure

The data file shall contain one record per row.  Each record is to be fully contained within one row.  Each record received will be assumed to be of the format specified in the “Data Record Format” subsection below.

At a minimum, this data file will contain an identical number of records as in the Load Forecast Data File, i.e. 288 records.  However, it is expected there will be multiple records for many if not every time point thus increasing the overall file length accordingly.  See the Appendix for an example of this data file.

The data records shall be provided in ascending chronological order.

### Data Record Format

Each data record for Hourly Dispatch data file shall contain the following fields beginning with the first column or character position of the record:

hour, tsr, MW

Comment lines are supported provided the “!” character is contained in the first column or character position of the record.  Comment lines are ignored by input processing.

The attributes for each record are defined in the table below:

| Field Name | Field Description | Format | Length | Units |
| --- | --- | --- | --- | --- |
| hour | Beginning hour date and time. The record will pertain to the first minute of the hour specified. | dd-mmm-yyyy-hh-mm | Fixed char(17) | Date &Time |
| tsr | Transmission Service Request number as identified on Entergy’s OASIS site with “confirmed” status. | Number | Variable int(8), e.g. 99999999 | Identifier |
| MW | MW dispatch from the specific TSR for this hour. Must be within the bounds of the TSR. | Number | Variable int(4), e.g. 9999 | MW |

Table 3 Hourly File Field Formats

## Unit Commitment Dispatch Option

The Network Customer may provide a list of designated long-term network resources to be used when balancing the projected load for that Network Customer.  Specific unit information consistent with Entergy Transmission’s EMS definitions and according to the format defined in this section should be provided when selecting this dispatch option.

The cumulative amount of unit dispatch for each time period is not expected to exceed the forecast load requirement from the corresponding hour or day in the Load Forecast data file.  When a short-fall exists, the Default Dispatch Process will be used to satisfy the remaining load requirement for that time point.  Some unit dispatch requests may not be included if the cumulative dispatch exceeds the forecast load requirement for any time point.

This option is compatible with the TSR Stack option where the Network Customer may provide both unit commitment information and a TSR stack data file.  Under this scenario, the unit dispatch will occur first, followed by use of the TSR stack to resolve any short-fall that may exist.  Once the TSR stack is exhausted and should a short-fall continue to exist, the Default Dispatch Process will be used to satisfy the remaining load requirement for that time point.  For this combined option, it is expected that the unit commitment data will reflect long-term network resources and the TSR stack will reflect short-term resources.

Validation checks will be performed to ensure that any combination of unit commitment and TSR information is compatible.

### Unit Commitment Data File Structure

The data file shall contain one record per row.  Each record is to be fully contained within one row.  Each record received will be assumed to be of the format specified in the “Data Record Format” subsection below.

At a minimum, this data file should contain an identical number of records as in the Load Forecast Data File, i.e. 288 records.  However, it is expected there will be multiple records for many if not every time point thus increasing the overall file length accordingly.  See the Appendix for an example of this data file.

The data records shall be provided in ascending chronological order.

### Unit Commitment Data Record Format

Each data record for Unit Commitment data file shall contain the following fields beginning with the first column or character position of the record:

hour, plant, unit, MW, status, min MW, max MW

Comment lines are supported provided the “!” character is contained in the first column or character position of the record.  Comment lines are ignored by input processing.

The attributes for each record are defined in the table below:

| Field Name | Field Description | Format | Length | Units |
| --- | --- | --- | --- | --- |
| hour | Beginning hour date and time. The record will pertain to the first minute of the hour specified. | dd-mmm-yyyy-hh-mm | Fixed char(17) | Date &Time |
| plant | Plant identifier as labeled within Entergy’s network model. | Alpha-numeric | Variable char(8) | Identifier |
| unit | Generator or Unit identifier as labeled within Entergy’s network model. | Alpha-numeric | Variable char(4) | Identifier |
| MW | MW dispatch for this hour. Must be within the bounds of Max and Min MW parameters for the given hour. | Number | Variable int(4), e.g. 9999 | MW |
| status | Unit availability for the given hour. “0” unit is not available; “1” unit is available. | Number | char(1) | Flag – “0” or “1” |
| min MW | Minimum MW dispatch for the unit during the given hour. Must be greater or equal to 0 and less than Max MW. | Number | Variable int(4), e.g. 9999 | MW |
| max MW | Maximum MW dispatch for the unit during the given hour. Must be greater than Min MW. | Number | Variable int(4), e.g. 9999 | MW |

Table 4 UC File Field Formats

# General Requirements

This section summarizes requirements that pertain to all data files and/or to the overall process.

Data files are required each business day.  A business day is considered to be Monday-Friday with the exception of federally recognized holidays.  This is the minimum requirement.  There are no restrictions against providing data files on weekends or holidays.

The Network Customer is expected to provide data for a time span of 35 days.  The first 11 days are to be provided in an hourly format, the remaining 24 days are to be provided in a daily format as defined for each data file type.  The duration of 35 days is requested to account for weekends, holidays and long weekends due to holidays observed on Friday or Monday.

The first day of the report is the current day.  For example, if the current day is Friday, June 1, 2007, then the data file would contain hourly records for 6/1/07 (day 1) through 6/11/07 (day 11).  The remainder of the file would contain daily records for 6/12/07 (day 12) through 7/5/07 (day 35).

Data files are expected by 9 A.M. Central Prevailing Time (CPT).  There is no restriction to the number of times a set of data files may be submitted during a given day.  The most recent data file will be assumed to be the most current.

Data files received by 9AM CPT on the Wednesday of each week will be used as input to the WPP process.

Only one dispatch option may be selected with each data file update.  The Network Customer may change the dispatch option desired with each data file update during the day or day to day.  The dispatch option will take affect with the next AFC hourly process.

All time references are of the “Hour Beginning” style.  The first hour of each day is the 60 minutes starting with the timestamp designed as “dd-mmm-yyyy-00-00” in the “dd-mmm-yyyy-hh-mm” format while the last hour of each day is the 60 minutes starting with the timestamp designated as “dd-mmm-yyyy-23-00”.

Any TSR provided must be a confirmed reservation on Entergy’s OASIS for network resources.  TSR’s will be processed in the order in which they appear in the data file.

All references to “load”, “load requirement”, “projected load” or other term includes load plus losses unless otherwise specifically stated that it does not include losses.

| Item | Description | Value |
| --- | --- | --- |
| file structure | All data files shall be ASCII encoded text in comma delimited format as is commonly available from Microsoft Excel and other products. | CSV |
| lf file name | The LF data file shall include the OASIS name of the Network Customer appended with “_LF” to indicate this is the load forecast file. This file is required each business day. | name_LF.CSV |
| stack file name | The stack data file shall include the OASIS name of the Network Customer appended with “_RES_STACK” to indicate this is the load forecast file. This file is optional each business day. | name_RES_STACK.CSV |
| hourly file name | The hourly data file shall include the OASIS name of the Network Customer appended with “_HRLY_DISPATCH” to indicate this is the hourly dispatch file. This file is optional each business day. | name_HRLY_DISPATCH.CSV |
| uc file name | The UC data file shall include the OASIS name of the Network Customer appended with “_UC” to indicate this is the unit commitment file. This file is optional each business day. | name_UC.CSV |

Table 5 General Requirements

# Implementation Milestones

The following table defines the important milestones in support of the full implementation of the AFC Reservation Dispatch Methodology.  Initially, the “Default” option will be available.  This is described as Phase A of the project implementation.  As Entergy currently supports both LF and UC data types today, the first phase of the implementation is to prepare the data file update website and implement the software changes necessary with the AFC process to support balancing load by Network Customer by fully utilizing Load Forecast data files and searching OASIS for appropriate reservations for network resources.  It is important that this portion is completed in order to support the WPP process scheduled near the same time as Phase A.

Later in 2007, full support of all of the proposed dispatch options, including use of reservation stacks, hourly listing of reservations, and combined unit commitment and hourly reservations will be provided.

| Feature | Phase A |  | Phase B |  |
| --- | --- | --- | --- | --- |
|  | Test | Go-Live | Test | Go-Live |
| LF Data File (Default Option) | Apr 2, 2007 | May 7, 2007 |  |  |
| UC Data File Option (Default) | Apr 2, 2007 | May 7, 2007 |  |  |
| TSR Stack Option |  |  | August Sept 4, 2007 | Oct 7, 2007 |
| TSR Hourly Option |  |  | Sept 4, 2007 | Oct 7, 2007 |
| Combined UC and TSR Stack Option |  |  | Sept 4, 2007 | Oct 7, 2007 |

Table 6 Implementation Milestones

# Appendix – Data File Examples

## Overview

This appendix contains examples of the file formats given in the body of this document.  Included are the following:

- Load Forecast (LF) Data File

- Unit Commitment (UC) Data File

- TSR Reservation Stack Data File

- TSR Hourly Dispatch Data File

These files were prepared to be consistent.  In other words, the UC, Hourly TSR and Stack TSR files reflect the time stamps and the load projections provided in the LF data file shown.

The following assumptions apply to these hypothetical examples:

- The Network Customer is Entergy (OASIS Name = ENTEMO)

- The load varies between 2290 and 3720 MW

- Day 1 is June 1, 2007

- 41 TSR’s are available for dispatch; sequenced 12345010…12345410; each representing 1500 MW

- Six fictitious generation units, named after Louisiana’s lakes, were available for dispatch as per the following table:

| Plant | Unit | Min MW | Max MW |
| --- | --- | --- | --- |
| BORGNE | G1 | 400 | 800 |
| PONTCHATRAIN | G2 | 600 | 1200 |
| MAUREPAS | G3 | 500 | 1000 |
| BORGNE | G4 | 300 | 600 |
| PONTCHATRAIN | G5 | 200 | 400 |
| MAUREPAS | G6 | 250 | 500 |

Table 7 Generation Unit Table

Please note:  The following sections contain full length (i.e. 35 days) examples of each data file type.  For simplicity and ease of printing, data for days Jun 2 through Jun 11 have been flagged in Microsoft Word as “hidden” text.  You may reveal this text using “Show/Hide Paragraph” button on the Microsoft Word Standard Toolbar.  See Microsoft Word Help “hidden text” for exact instructions.

## Load Forecast Data File Example

The Load Forecast file should contain 288 records as demonstrated in the example below.  The first 264 records provide an hourly peak MW value for the current day (day 1) through day 11.  The remaining 24 records indicate daily peak MW projections for the next 24 days (day 12 through day 35).

Note that the time stamp for the daily peaks should correspond to the hour when the peak for that Network Customer is expected.  For simplicity sake in preparing this example, the peak hour for the first 11 days is nominally 14-00.  The peak hour for the remaining days varies between 14-00 and 16-00 to demonstrate that the peak hour may not be the same time each day.

(Note: Day 2 through Day 11 may be revealed using “Show/Hide Paragraph” button on Microsoft Word Standard Toolbar.)

### Data File Beginning with Record 1:

01-JUN-2007-00-00,ENTEMO,2305

01-JUN-2007-01-00,ENTEMO,2295

01-JUN-2007-02-00,ENTEMO,2290

01-JUN-2007-03-00,ENTEMO,2295

01-JUN-2007-04-00,ENTEMO,2305

01-JUN-2007-05-00,ENTEMO,2325

01-JUN-2007-06-00,ENTEMO,2350

01-JUN-2007-07-00,ENTEMO,2380

01-JUN-2007-08-00,ENTEMO,2410

01-JUN-2007-09-00,ENTEMO,2440

01-JUN-2007-10-00,ENTEMO,2470

01-JUN-2007-11-00,ENTEMO,2495

01-JUN-2007-12-00,ENTEMO,2515

01-JUN-2007-13-00,ENTEMO,2525

01-JUN-2007-14-00,ENTEMO,2530

01-JUN-2007-15-00,ENTEMO,2525

01-JUN-2007-16-00,ENTEMO,2515

01-JUN-2007-17-00,ENTEMO,2495

01-JUN-2007-18-00,ENTEMO,2470

01-JUN-2007-19-00,ENTEMO,2440

01-JUN-2007-20-00,ENTEMO,2410

01-JUN-2007-21-00,ENTEMO,2380

01-JUN-2007-22-00,ENTEMO,2350

01-JUN-2007-23-00,ENTEMO,2325

02-JUN-2007-00-00,ENTEMO,2345

02-JUN-2007-01-00,ENTEMO,2335

02-JUN-2007-02-00,ENTEMO,2330

02-JUN-2007-03-00,ENTEMO,2335

02-JUN-2007-04-00,ENTEMO,2345

02-JUN-2007-05-00,ENTEMO,2365

02-JUN-2007-06-00,ENTEMO,2390

02-JUN-2007-07-00,ENTEMO,2420

02-JUN-2007-08-00,ENTEMO,2450

02-JUN-2007-09-00,ENTEMO,2480

02-JUN-2007-10-00,ENTEMO,2510

02-JUN-2007-11-00,ENTEMO,2535

02-JUN-2007-12-00,ENTEMO,2555

02-JUN-2007-13-00,ENTEMO,2565

02-JUN-2007-14-00,ENTEMO,2570

02-JUN-2007-15-00,ENTEMO,2565

02-JUN-2007-16-00,ENTEMO,2555

02-JUN-2007-17-00,ENTEMO,2535

02-JUN-2007-18-00,ENTEMO,2510

02-JUN-2007-19-00,ENTEMO,2480

02-JUN-2007-20-00,ENTEMO,2450

02-JUN-2007-21-00,ENTEMO,2420

02-JUN-2007-22-00,ENTEMO,2390

02-JUN-2007-23-00,ENTEMO,2365

03-JUN-2007-00-00,ENTEMO,2410

03-JUN-2007-01-00,ENTEMO,2395

03-JUN-2007-02-00,ENTEMO,2395

03-JUN-2007-03-00,ENTEMO,2395

03-JUN-2007-04-00,ENTEMO,2410

03-JUN-2007-05-00,ENTEMO,2430

03-JUN-2007-06-00,ENTEMO,2455

03-JUN-2007-07-00,ENTEMO,2480

03-JUN-2007-08-00,ENTEMO,2515

03-JUN-2007-09-00,ENTEMO,2545

03-JUN-2007-10-00,ENTEMO,2575

03-JUN-2007-11-00,ENTEMO,2600

03-JUN-2007-12-00,ENTEMO,2615

03-JUN-2007-13-00,ENTEMO,2630

03-JUN-2007-14-00,ENTEMO,2635

03-JUN-2007-15-00,ENTEMO,2630

03-JUN-2007-16-00,ENTEMO,2615

03-JUN-2007-17-00,ENTEMO,2600

03-JUN-2007-18-00,ENTEMO,2575

03-JUN-2007-19-00,ENTEMO,2545

03-JUN-2007-20-00,ENTEMO,2515

03-JUN-2007-21-00,ENTEMO,2480

03-JUN-2007-22-00,ENTEMO,2455

03-JUN-2007-23-00,ENTEMO,2430

04-JUN-2007-00-00,ENTEMO,2495

04-JUN-2007-01-00,ENTEMO,2480

04-JUN-2007-02-00,ENTEMO,2475

04-JUN-2007-03-00,ENTEMO,2480

04-JUN-2007-04-00,ENTEMO,2495

04-JUN-2007-05-00,ENTEMO,2510

04-JUN-2007-06-00,ENTEMO,2535

04-JUN-2007-07-00,ENTEMO,2565

04-JUN-2007-08-00,ENTEMO,2595

04-JUN-2007-09-00,ENTEMO,2630

04-JUN-2007-10-00,ENTEMO,2655

04-JUN-2007-11-00,ENTEMO,2680

04-JUN-2007-12-00,ENTEMO,2700

04-JUN-2007-13-00,ENTEMO,2715

04-JUN-2007-14-00,ENTEMO,2715

04-JUN-2007-15-00,ENTEMO,2715

04-JUN-2007-16-00,ENTEMO,2700

04-JUN-2007-17-00,ENTEMO,2680

04-JUN-2007-18-00,ENTEMO,2655

04-JUN-2007-19-00,ENTEMO,2630

04-JUN-2007-20-00,ENTEMO,2595

04-JUN-2007-21-00,ENTEMO,2565

04-JUN-2007-22-00,ENTEMO,2535

04-JUN-2007-23-00,ENTEMO,2510

05-JUN-2007-00-00,ENTEMO,2595

05-JUN-2007-01-00,ENTEMO,2580

05-JUN-2007-02-00,ENTEMO,2580

05-JUN-2007-03-00,ENTEMO,2580

05-JUN-2007-04-00,ENTEMO,2595

05-JUN-2007-05-00,ENTEMO,2615

05-JUN-2007-06-00,ENTEMO,2640

05-JUN-2007-07-00,ENTEMO,2665

05-JUN-2007-08-00,ENTEMO,2700

05-JUN-2007-09-00,ENTEMO,2730

05-JUN-2007-10-00,ENTEMO,2760

05-JUN-2007-11-00,ENTEMO,2785

05-JUN-2007-12-00,ENTEMO,2800

05-JUN-2007-13-00,ENTEMO,2815

05-JUN-2007-14-00,ENTEMO,2820

05-JUN-2007-15-00,ENTEMO,2815

05-JUN-2007-16-00,ENTEMO,2800

05-JUN-2007-17-00,ENTEMO,2785

05-JUN-2007-18-00,ENTEMO,2760

05-JUN-2007-19-00,ENTEMO,2730

05-JUN-2007-20-00,ENTEMO,2700

05-JUN-2007-21-00,ENTEMO,2665

05-JUN-2007-22-00,ENTEMO,2640

05-JUN-2007-23-00,ENTEMO,2615

06-JUN-2007-00-00,ENTEMO,2710

06-JUN-2007-01-00,ENTEMO,2695

06-JUN-2007-02-00,ENTEMO,2695

06-JUN-2007-03-00,ENTEMO,2695

06-JUN-2007-04-00,ENTEMO,2710

06-JUN-2007-05-00,ENTEMO,2730

06-JUN-2007-06-00,ENTEMO,2755

06-JUN-2007-07-00,ENTEMO,2780

06-JUN-2007-08-00,ENTEMO,2815

06-JUN-2007-09-00,ENTEMO,2845

06-JUN-2007-10-00,ENTEMO,2875

06-JUN-2007-11-00,ENTEMO,2900

06-JUN-2007-12-00,ENTEMO,2915

06-JUN-2007-13-00,ENTEMO,2930

06-JUN-2007-14-00,ENTEMO,2935

06-JUN-2007-15-00,ENTEMO,2930

06-JUN-2007-16-00,ENTEMO,2915

06-JUN-2007-17-00,ENTEMO,2900

06-JUN-2007-18-00,ENTEMO,2875

06-JUN-2007-19-00,ENTEMO,2845

06-JUN-2007-20-00,ENTEMO,2815

06-JUN-2007-21-00,ENTEMO,2780

06-JUN-2007-22-00,ENTEMO,2755

06-JUN-2007-23-00,ENTEMO,2730

07-JUN-2007-00-00,ENTEMO,2830

07-JUN-2007-01-00,ENTEMO,2820

07-JUN-2007-02-00,ENTEMO,2815

07-JUN-2007-03-00,ENTEMO,2820

07-JUN-2007-04-00,ENTEMO,2830

07-JUN-2007-05-00,ENTEMO,2850

07-JUN-2007-06-00,ENTEMO,2875

07-JUN-2007-07-00,ENTEMO,2905

07-JUN-2007-08-00,ENTEMO,2935

07-JUN-2007-09-00,ENTEMO,2965

07-JUN-2007-10-00,ENTEMO,2995

07-JUN-2007-11-00,ENTEMO,3020

07-JUN-2007-12-00,ENTEMO,3040

07-JUN-2007-13-00,ENTEMO,3050

07-JUN-2007-14-00,ENTEMO,3055

07-JUN-2007-15-00,ENTEMO,3050

07-JUN-2007-16-00,ENTEMO,3040

07-JUN-2007-17-00,ENTEMO,3020

07-JUN-2007-18-00,ENTEMO,2995

07-JUN-2007-19-00,ENTEMO,2965

07-JUN-2007-20-00,ENTEMO,2935

07-JUN-2007-21-00,ENTEMO,2905

07-JUN-2007-22-00,ENTEMO,2875

07-JUN-2007-23-00,ENTEMO,2850

08-JUN-2007-00-00,ENTEMO,2955

08-JUN-2007-01-00,ENTEMO,2945

08-JUN-2007-02-00,ENTEMO,2940

08-JUN-2007-03-00,ENTEMO,2945

08-JUN-2007-04-00,ENTEMO,2955

08-JUN-2007-05-00,ENTEMO,2975

08-JUN-2007-06-00,ENTEMO,3000

08-JUN-2007-07-00,ENTEMO,3030

08-JUN-2007-08-00,ENTEMO,3060

08-JUN-2007-09-00,ENTEMO,3090

08-JUN-2007-10-00,ENTEMO,3120

08-JUN-2007-11-00,ENTEMO,3145

08-JUN-2007-12-00,ENTEMO,3165

08-JUN-2007-13-00,ENTEMO,3175

08-JUN-2007-14-00,ENTEMO,3180

08-JUN-2007-15-00,ENTEMO,3175

08-JUN-2007-16-00,ENTEMO,3165

08-JUN-2007-17-00,ENTEMO,3145

08-JUN-2007-18-00,ENTEMO,3120

08-JUN-2007-19-00,ENTEMO,3090

08-JUN-2007-20-00,ENTEMO,3060

08-JUN-2007-21-00,ENTEMO,3030

08-JUN-2007-22-00,ENTEMO,3000

08-JUN-2007-23-00,ENTEMO,2975

09-JUN-2007-00-00,ENTEMO,3080

09-JUN-2007-01-00,ENTEMO,3065

09-JUN-2007-02-00,ENTEMO,3065

09-JUN-2007-03-00,ENTEMO,3065

09-JUN-2007-04-00,ENTEMO,3080

09-JUN-2007-05-00,ENTEMO,3100

09-JUN-2007-06-00,ENTEMO,3125

09-JUN-2007-07-00,ENTEMO,3150

09-JUN-2007-08-00,ENTEMO,3185

09-JUN-2007-09-00,ENTEMO,3215

09-JUN-2007-10-00,ENTEMO,3245

09-JUN-2007-11-00,ENTEMO,3270

09-JUN-2007-12-00,ENTEMO,3285

09-JUN-2007-13-00,ENTEMO,3300

09-JUN-2007-14-00,ENTEMO,3305

09-JUN-2007-15-00,ENTEMO,3300

09-JUN-2007-16-00,ENTEMO,3285

09-JUN-2007-17-00,ENTEMO,3270

09-JUN-2007-18-00,ENTEMO,3245

09-JUN-2007-19-00,ENTEMO,3215

09-JUN-2007-20-00,ENTEMO,3185

09-JUN-2007-21-00,ENTEMO,3150

09-JUN-2007-22-00,ENTEMO,3125

09-JUN-2007-23-00,ENTEMO,3100

10-JUN-2007-00-00,ENTEMO,3195

10-JUN-2007-01-00,ENTEMO,3180

10-JUN-2007-02-00,ENTEMO,3180

10-JUN-2007-03-00,ENTEMO,3180

10-JUN-2007-04-00,ENTEMO,3195

10-JUN-2007-05-00,ENTEMO,3215

10-JUN-2007-06-00,ENTEMO,3240

10-JUN-2007-07-00,ENTEMO,3265

10-JUN-2007-08-00,ENTEMO,3300

10-JUN-2007-09-00,ENTEMO,3330

10-JUN-2007-10-00,ENTEMO,3360

10-JUN-2007-11-00,ENTEMO,3385

10-JUN-2007-12-00,ENTEMO,3400

10-JUN-2007-13-00,ENTEMO,3415

10-JUN-2007-14-00,ENTEMO,3420

10-JUN-2007-15-00,ENTEMO,3415

10-JUN-2007-16-00,ENTEMO,3400

10-JUN-2007-17-00,ENTEMO,3385

10-JUN-2007-18-00,ENTEMO,3360

10-JUN-2007-19-00,ENTEMO,3330

10-JUN-2007-20-00,ENTEMO,3300

10-JUN-2007-21-00,ENTEMO,3265

10-JUN-2007-22-00,ENTEMO,3240

10-JUN-2007-23-00,ENTEMO,3215

11-JUN-2007-00-00,ENTEMO,3295

11-JUN-2007-01-00,ENTEMO,3285

11-JUN-2007-02-00,ENTEMO,3280

11-JUN-2007-03-00,ENTEMO,3285

11-JUN-2007-04-00,ENTEMO,3295

11-JUN-2007-05-00,ENTEMO,3315

11-JUN-2007-06-00,ENTEMO,3340

11-JUN-2007-07-00,ENTEMO,3370

11-JUN-2007-08-00,ENTEMO,3400

11-JUN-2007-09-00,ENTEMO,3430

11-JUN-2007-10-00,ENTEMO,3460

11-JUN-2007-11-00,ENTEMO,3485

11-JUN-2007-12-00,ENTEMO,3505

11-JUN-2007-13-00,ENTEMO,3515

11-JUN-2007-14-00,ENTEMO,3520

11-JUN-2007-15-00,ENTEMO,3515

11-JUN-2007-16-00,ENTEMO,3505

11-JUN-2007-17-00,ENTEMO,3485

11-JUN-2007-18-00,ENTEMO,3460

11-JUN-2007-19-00,ENTEMO,3430

11-JUN-2007-20-00,ENTEMO,3400

11-JUN-2007-21-00,ENTEMO,3370

11-JUN-2007-22-00,ENTEMO,3340

11-JUN-2007-23-00,ENTEMO,3315

12-JUN-2007-14-00,ENTEMO,3605

13-JUN-2007-15-00,ENTEMO,3660

14-JUN-2007-16-00,ENTEMO,3690

15-JUN-2007-14-00,ENTEMO,3720

16-JUN-2007-15-00,ENTEMO,3700

17-JUN-2007-16-00,ENTEMO,3650

18-JUN-2007-14-00,ENTEMO,3605

19-JUN-2007-15-00,ENTEMO,3515

20-JUN-2007-16-00,ENTEMO,3400

21-JUN-2007-14-00,ENTEMO,3305

22-JUN-2007-15-00,ENTEMO,3175

23-JUN-2007-16-00,ENTEMO,3040

24-JUN-2007-14-00,ENTEMO,2935

25-JUN-2007-15-00,ENTEMO,2815

26-JUN-2007-16-00,ENTEMO,2700

27-JUN-2007-14-00,ENTEMO,2635

28-JUN-2007-15-00,ENTEMO,2565

29-JUN-2007-16-00,ENTEMO,2515

30-JUN-2007-14-00,ENTEMO,2520

01-JUL-2007-15-00,ENTEMO,2525

02-JUL-2007-16-00,ENTEMO,2555

03-JUL-2007-14-00,ENTEMO,2635

04-JUL-2007-15-00,ENTEMO,2715

05-JUL-2007-16-00,ENTEMO,2800

## Unit Commitment Data File Example

The following UC Data File example matches the projected load provided in the corresponding LF Data File in this appendix.  Units were dispatched in the order provided in the Generation Unit Table listed under the Overview section of this appendix.

In this example, units were chosen for dispatch at no more than 85% of the assigned Max MW value.  This percentage was arbitrarily chosen for illustrative purposes of this example.  Units were constrained to run at least at the designated Min MW value.  A careful cross check will show some time points do not exactly match the associated load projection in the Load Forecast data file.  This is expected in actual data files and the short fall will be resolved by either the default processing or use of the Combined UC option.

Note that while the Combined UC option allows for inclusion of a TSR Stack Data File with the UC Data File to fulfill the obligation to meet the projected MW load, this example assumes the full dispatch is provided in the UC Data File.

(Note: Day 2 through Day 11 may be revealed using “Show/Hide Paragraph” button on Microsoft Word Standard Toolbar.)

### Data File Beginning with Record 1:

01-JUN-2007-00-00,BORGNE,G1,680,1,400,800

01-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-00-00,MAUREPAS,G3,605,1,500,1000

01-JUN-2007-01-00,BORGNE,G1,680,1,400,800

01-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-01-00,MAUREPAS,G3,595,1,500,1000

01-JUN-2007-02-00,BORGNE,G1,680,1,400,800

01-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-02-00,MAUREPAS,G3,590,1,500,1000

01-JUN-2007-03-00,BORGNE,G1,680,1,400,800

01-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-03-00,MAUREPAS,G3,595,1,500,1000

01-JUN-2007-04-00,BORGNE,G1,680,1,400,800

01-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-04-00,MAUREPAS,G3,605,1,500,1000

01-JUN-2007-05-00,BORGNE,G1,680,1,400,800

01-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-05-00,MAUREPAS,G3,625,1,500,1000

01-JUN-2007-06-00,BORGNE,G1,680,1,400,800

01-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-06-00,MAUREPAS,G3,650,1,500,1000

01-JUN-2007-07-00,BORGNE,G1,680,1,400,800

01-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-07-00,MAUREPAS,G3,680,1,500,1000

01-JUN-2007-08-00,BORGNE,G1,680,1,400,800

01-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-08-00,MAUREPAS,G3,710,1,500,1000

01-JUN-2007-09-00,BORGNE,G1,680,1,400,800

01-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-09-00,MAUREPAS,G3,740,1,500,1000

01-JUN-2007-10-00,BORGNE,G1,680,1,400,800

01-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-10-00,MAUREPAS,G3,770,1,500,1000

01-JUN-2007-11-00,BORGNE,G1,680,1,400,800

01-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-11-00,MAUREPAS,G3,795,1,500,1000

01-JUN-2007-12-00,BORGNE,G1,680,1,400,800

01-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-12-00,MAUREPAS,G3,815,1,500,1000

01-JUN-2007-13-00,BORGNE,G1,680,1,400,800

01-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-13-00,MAUREPAS,G3,825,1,500,1000

01-JUN-2007-14-00,BORGNE,G1,680,1,400,800

01-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-14-00,MAUREPAS,G3,830,1,500,1000

01-JUN-2007-15-00,BORGNE,G1,680,1,400,800

01-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-15-00,MAUREPAS,G3,825,1,500,1000

01-JUN-2007-16-00,BORGNE,G1,680,1,400,800

01-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-16-00,MAUREPAS,G3,815,1,500,1000

01-JUN-2007-17-00,BORGNE,G1,680,1,400,800

01-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-17-00,MAUREPAS,G3,795,1,500,1000

01-JUN-2007-18-00,BORGNE,G1,680,1,400,800

01-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-18-00,MAUREPAS,G3,770,1,500,1000

01-JUN-2007-19-00,BORGNE,G1,680,1,400,800

01-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-19-00,MAUREPAS,G3,740,1,500,1000

01-JUN-2007-20-00,BORGNE,G1,680,1,400,800

01-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-20-00,MAUREPAS,G3,710,1,500,1000

01-JUN-2007-21-00,BORGNE,G1,680,1,400,800

01-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-21-00,MAUREPAS,G3,680,1,500,1000

01-JUN-2007-22-00,BORGNE,G1,680,1,400,800

01-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-22-00,MAUREPAS,G3,650,1,500,1000

01-JUN-2007-23-00,BORGNE,G1,680,1,400,800

01-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUN-2007-23-00,MAUREPAS,G3,625,1,500,1000

02-JUN-2007-00-00,BORGNE,G1,680,1,400,800

02-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-00-00,MAUREPAS,G3,645,1,500,1000

02-JUN-2007-01-00,BORGNE,G1,680,1,400,800

02-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-01-00,MAUREPAS,G3,635,1,500,1000

02-JUN-2007-02-00,BORGNE,G1,680,1,400,800

02-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-02-00,MAUREPAS,G3,630,1,500,1000

02-JUN-2007-03-00,BORGNE,G1,680,1,400,800

02-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-03-00,MAUREPAS,G3,635,1,500,1000

02-JUN-2007-04-00,BORGNE,G1,680,1,400,800

02-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-04-00,MAUREPAS,G3,645,1,500,1000

02-JUN-2007-05-00,BORGNE,G1,680,1,400,800

02-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-05-00,MAUREPAS,G3,665,1,500,1000

02-JUN-2007-06-00,BORGNE,G1,680,1,400,800

02-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-06-00,MAUREPAS,G3,690,1,500,1000

02-JUN-2007-07-00,BORGNE,G1,680,1,400,800

02-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-07-00,MAUREPAS,G3,720,1,500,1000

02-JUN-2007-08-00,BORGNE,G1,680,1,400,800

02-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-08-00,MAUREPAS,G3,750,1,500,1000

02-JUN-2007-09-00,BORGNE,G1,680,1,400,800

02-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-09-00,MAUREPAS,G3,780,1,500,1000

02-JUN-2007-10-00,BORGNE,G1,680,1,400,800

02-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-10-00,MAUREPAS,G3,810,1,500,1000

02-JUN-2007-11-00,BORGNE,G1,680,1,400,800

02-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-11-00,MAUREPAS,G3,835,1,500,1000

02-JUN-2007-12-00,BORGNE,G1,680,1,400,800

02-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

02-JUN-2007-13-00,BORGNE,G1,680,1,400,800

02-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

02-JUN-2007-14-00,BORGNE,G1,680,1,400,800

02-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

02-JUN-2007-15-00,BORGNE,G1,680,1,400,800

02-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

02-JUN-2007-16-00,BORGNE,G1,680,1,400,800

02-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

02-JUN-2007-17-00,BORGNE,G1,680,1,400,800

02-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-17-00,MAUREPAS,G3,835,1,500,1000

02-JUN-2007-18-00,BORGNE,G1,680,1,400,800

02-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-18-00,MAUREPAS,G3,810,1,500,1000

02-JUN-2007-19-00,BORGNE,G1,680,1,400,800

02-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-19-00,MAUREPAS,G3,780,1,500,1000

02-JUN-2007-20-00,BORGNE,G1,680,1,400,800

02-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-20-00,MAUREPAS,G3,750,1,500,1000

02-JUN-2007-21-00,BORGNE,G1,680,1,400,800

02-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-21-00,MAUREPAS,G3,720,1,500,1000

02-JUN-2007-22-00,BORGNE,G1,680,1,400,800

02-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-22-00,MAUREPAS,G3,690,1,500,1000

02-JUN-2007-23-00,BORGNE,G1,680,1,400,800

02-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUN-2007-23-00,MAUREPAS,G3,665,1,500,1000

03-JUN-2007-00-00,BORGNE,G1,680,1,400,800

03-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-00-00,MAUREPAS,G3,710,1,500,1000

03-JUN-2007-01-00,BORGNE,G1,680,1,400,800

03-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-01-00,MAUREPAS,G3,695,1,500,1000

03-JUN-2007-02-00,BORGNE,G1,680,1,400,800

03-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-02-00,MAUREPAS,G3,695,1,500,1000

03-JUN-2007-03-00,BORGNE,G1,680,1,400,800

03-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-03-00,MAUREPAS,G3,695,1,500,1000

03-JUN-2007-04-00,BORGNE,G1,680,1,400,800

03-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-04-00,MAUREPAS,G3,710,1,500,1000

03-JUN-2007-05-00,BORGNE,G1,680,1,400,800

03-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-05-00,MAUREPAS,G3,730,1,500,1000

03-JUN-2007-06-00,BORGNE,G1,680,1,400,800

03-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-06-00,MAUREPAS,G3,755,1,500,1000

03-JUN-2007-07-00,BORGNE,G1,680,1,400,800

03-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-07-00,MAUREPAS,G3,780,1,500,1000

03-JUN-2007-08-00,BORGNE,G1,680,1,400,800

03-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-08-00,MAUREPAS,G3,815,1,500,1000

03-JUN-2007-09-00,BORGNE,G1,680,1,400,800

03-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-09-00,MAUREPAS,G3,845,1,500,1000

03-JUN-2007-10-00,BORGNE,G1,680,1,400,800

03-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-11-00,BORGNE,G1,680,1,400,800

03-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-12-00,BORGNE,G1,680,1,400,800

03-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-13-00,BORGNE,G1,680,1,400,800

03-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-14-00,BORGNE,G1,680,1,400,800

03-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-15-00,BORGNE,G1,680,1,400,800

03-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-16-00,BORGNE,G1,680,1,400,800

03-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-17-00,BORGNE,G1,680,1,400,800

03-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-18-00,BORGNE,G1,680,1,400,800

03-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

03-JUN-2007-19-00,BORGNE,G1,680,1,400,800

03-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-19-00,MAUREPAS,G3,845,1,500,1000

03-JUN-2007-20-00,BORGNE,G1,680,1,400,800

03-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-20-00,MAUREPAS,G3,815,1,500,1000

03-JUN-2007-21-00,BORGNE,G1,680,1,400,800

03-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-21-00,MAUREPAS,G3,780,1,500,1000

03-JUN-2007-22-00,BORGNE,G1,680,1,400,800

03-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-22-00,MAUREPAS,G3,755,1,500,1000

03-JUN-2007-23-00,BORGNE,G1,680,1,400,800

03-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUN-2007-23-00,MAUREPAS,G3,730,1,500,1000

04-JUN-2007-00-00,BORGNE,G1,680,1,400,800

04-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-00-00,MAUREPAS,G3,795,1,500,1000

04-JUN-2007-01-00,BORGNE,G1,680,1,400,800

04-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-01-00,MAUREPAS,G3,780,1,500,1000

04-JUN-2007-02-00,BORGNE,G1,680,1,400,800

04-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-02-00,MAUREPAS,G3,775,1,500,1000

04-JUN-2007-03-00,BORGNE,G1,680,1,400,800

04-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-03-00,MAUREPAS,G3,780,1,500,1000

04-JUN-2007-04-00,BORGNE,G1,680,1,400,800

04-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-04-00,MAUREPAS,G3,795,1,500,1000

04-JUN-2007-05-00,BORGNE,G1,680,1,400,800

04-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-05-00,MAUREPAS,G3,810,1,500,1000

04-JUN-2007-06-00,BORGNE,G1,680,1,400,800

04-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-06-00,MAUREPAS,G3,835,1,500,1000

04-JUN-2007-07-00,BORGNE,G1,680,1,400,800

04-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-08-00,BORGNE,G1,680,1,400,800

04-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-09-00,BORGNE,G1,680,1,400,800

04-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-10-00,BORGNE,G1,680,1,400,800

04-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-11-00,BORGNE,G1,680,1,400,800

04-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-12-00,BORGNE,G1,680,1,400,800

04-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-13-00,BORGNE,G1,680,1,400,800

04-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-14-00,BORGNE,G1,680,1,400,800

04-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-15-00,BORGNE,G1,680,1,400,800

04-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-16-00,BORGNE,G1,680,1,400,800

04-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-17-00,BORGNE,G1,680,1,400,800

04-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-18-00,BORGNE,G1,680,1,400,800

04-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-19-00,BORGNE,G1,680,1,400,800

04-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-20-00,BORGNE,G1,680,1,400,800

04-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-21-00,BORGNE,G1,680,1,400,800

04-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

04-JUN-2007-22-00,BORGNE,G1,680,1,400,800

04-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-22-00,MAUREPAS,G3,835,1,500,1000

04-JUN-2007-23-00,BORGNE,G1,680,1,400,800

04-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUN-2007-23-00,MAUREPAS,G3,810,1,500,1000

05-JUN-2007-00-00,BORGNE,G1,680,1,400,800

05-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-01-00,BORGNE,G1,680,1,400,800

05-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-02-00,BORGNE,G1,680,1,400,800

05-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-03-00,BORGNE,G1,680,1,400,800

05-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-04-00,BORGNE,G1,680,1,400,800

05-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-05-00,BORGNE,G1,680,1,400,800

05-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-06-00,BORGNE,G1,680,1,400,800

05-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-07-00,BORGNE,G1,680,1,400,800

05-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-08-00,BORGNE,G1,680,1,400,800

05-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-09-00,BORGNE,G1,680,1,400,800

05-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-10-00,BORGNE,G1,680,1,400,800

05-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-11-00,BORGNE,G1,680,1,400,800

05-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-12-00,BORGNE,G1,680,1,400,800

05-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-13-00,BORGNE,G1,680,1,400,800

05-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-14-00,BORGNE,G1,680,1,400,800

05-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-15-00,BORGNE,G1,680,1,400,800

05-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-16-00,BORGNE,G1,680,1,400,800

05-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-17-00,BORGNE,G1,680,1,400,800

05-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-18-00,BORGNE,G1,680,1,400,800

05-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-19-00,BORGNE,G1,680,1,400,800

05-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-20-00,BORGNE,G1,680,1,400,800

05-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-21-00,BORGNE,G1,680,1,400,800

05-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-22-00,BORGNE,G1,680,1,400,800

05-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

05-JUN-2007-23-00,BORGNE,G1,680,1,400,800

05-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-00-00,BORGNE,G1,680,1,400,800

06-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-01-00,BORGNE,G1,680,1,400,800

06-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-02-00,BORGNE,G1,680,1,400,800

06-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-03-00,BORGNE,G1,680,1,400,800

06-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-04-00,BORGNE,G1,680,1,400,800

06-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-05-00,BORGNE,G1,680,1,400,800

06-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-06-00,BORGNE,G1,680,1,400,800

06-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-07-00,BORGNE,G1,680,1,400,800

06-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-08-00,BORGNE,G1,680,1,400,800

06-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-09-00,BORGNE,G1,680,1,400,800

06-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-10-00,BORGNE,G1,680,1,400,800

06-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-10-00,BORGNE,G4,325,1,300,600

06-JUN-2007-11-00,BORGNE,G1,680,1,400,800

06-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-11-00,BORGNE,G4,350,1,300,600

06-JUN-2007-12-00,BORGNE,G1,680,1,400,800

06-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-12-00,BORGNE,G4,365,1,300,600

06-JUN-2007-13-00,BORGNE,G1,680,1,400,800

06-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-13-00,BORGNE,G4,380,1,300,600

06-JUN-2007-14-00,BORGNE,G1,680,1,400,800

06-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-14-00,BORGNE,G4,385,1,300,600

06-JUN-2007-15-00,BORGNE,G1,680,1,400,800

06-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-15-00,BORGNE,G4,380,1,300,600

06-JUN-2007-16-00,BORGNE,G1,680,1,400,800

06-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-16-00,BORGNE,G4,365,1,300,600

06-JUN-2007-17-00,BORGNE,G1,680,1,400,800

06-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-17-00,BORGNE,G4,350,1,300,600

06-JUN-2007-18-00,BORGNE,G1,680,1,400,800

06-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-18-00,BORGNE,G4,325,1,300,600

06-JUN-2007-19-00,BORGNE,G1,680,1,400,800

06-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-20-00,BORGNE,G1,680,1,400,800

06-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-21-00,BORGNE,G1,680,1,400,800

06-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-22-00,BORGNE,G1,680,1,400,800

06-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

06-JUN-2007-23-00,BORGNE,G1,680,1,400,800

06-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

06-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-00-00,BORGNE,G1,680,1,400,800

07-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-01-00,BORGNE,G1,680,1,400,800

07-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-02-00,BORGNE,G1,680,1,400,800

07-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-03-00,BORGNE,G1,680,1,400,800

07-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-04-00,BORGNE,G1,680,1,400,800

07-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-05-00,BORGNE,G1,680,1,400,800

07-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-05-00,BORGNE,G4,300,1,300,600

07-JUN-2007-06-00,BORGNE,G1,680,1,400,800

07-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-06-00,BORGNE,G4,325,1,300,600

07-JUN-2007-07-00,BORGNE,G1,680,1,400,800

07-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-07-00,BORGNE,G4,355,1,300,600

07-JUN-2007-08-00,BORGNE,G1,680,1,400,800

07-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-08-00,BORGNE,G4,385,1,300,600

07-JUN-2007-09-00,BORGNE,G1,680,1,400,800

07-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-09-00,BORGNE,G4,415,1,300,600

07-JUN-2007-10-00,BORGNE,G1,680,1,400,800

07-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-10-00,BORGNE,G4,445,1,300,600

07-JUN-2007-11-00,BORGNE,G1,680,1,400,800

07-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-11-00,BORGNE,G4,470,1,300,600

07-JUN-2007-12-00,BORGNE,G1,680,1,400,800

07-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-12-00,BORGNE,G4,490,1,300,600

07-JUN-2007-13-00,BORGNE,G1,680,1,400,800

07-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-13-00,BORGNE,G4,500,1,300,600

07-JUN-2007-14-00,BORGNE,G1,680,1,400,800

07-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-14-00,BORGNE,G4,505,1,300,600

07-JUN-2007-15-00,BORGNE,G1,680,1,400,800

07-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-15-00,BORGNE,G4,500,1,300,600

07-JUN-2007-16-00,BORGNE,G1,680,1,400,800

07-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-16-00,BORGNE,G4,490,1,300,600

07-JUN-2007-17-00,BORGNE,G1,680,1,400,800

07-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-17-00,BORGNE,G4,470,1,300,600

07-JUN-2007-18-00,BORGNE,G1,680,1,400,800

07-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-18-00,BORGNE,G4,445,1,300,600

07-JUN-2007-19-00,BORGNE,G1,680,1,400,800

07-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-19-00,BORGNE,G4,415,1,300,600

07-JUN-2007-20-00,BORGNE,G1,680,1,400,800

07-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-20-00,BORGNE,G4,385,1,300,600

07-JUN-2007-21-00,BORGNE,G1,680,1,400,800

07-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-21-00,BORGNE,G4,355,1,300,600

07-JUN-2007-22-00,BORGNE,G1,680,1,400,800

07-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-22-00,BORGNE,G4,325,1,300,600

07-JUN-2007-23-00,BORGNE,G1,680,1,400,800

07-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

07-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

07-JUN-2007-23-00,BORGNE,G4,300,1,300,600

08-JUN-2007-00-00,BORGNE,G1,680,1,400,800

08-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-00-00,BORGNE,G4,405,1,300,600

08-JUN-2007-01-00,BORGNE,G1,680,1,400,800

08-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-01-00,BORGNE,G4,395,1,300,600

08-JUN-2007-02-00,BORGNE,G1,680,1,400,800

08-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-02-00,BORGNE,G4,390,1,300,600

08-JUN-2007-03-00,BORGNE,G1,680,1,400,800

08-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-03-00,BORGNE,G4,395,1,300,600

08-JUN-2007-04-00,BORGNE,G1,680,1,400,800

08-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-04-00,BORGNE,G4,405,1,300,600

08-JUN-2007-05-00,BORGNE,G1,680,1,400,800

08-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-05-00,BORGNE,G4,425,1,300,600

08-JUN-2007-06-00,BORGNE,G1,680,1,400,800

08-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-06-00,BORGNE,G4,450,1,300,600

08-JUN-2007-07-00,BORGNE,G1,680,1,400,800

08-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-07-00,BORGNE,G4,480,1,300,600

08-JUN-2007-08-00,BORGNE,G1,680,1,400,800

08-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-08-00,BORGNE,G4,510,1,300,600

08-JUN-2007-09-00,BORGNE,G1,680,1,400,800

08-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-09-00,BORGNE,G4,510,1,300,600

08-JUN-2007-10-00,BORGNE,G1,680,1,400,800

08-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-10-00,BORGNE,G4,510,1,300,600

08-JUN-2007-11-00,BORGNE,G1,680,1,400,800

08-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-11-00,BORGNE,G4,510,1,300,600

08-JUN-2007-12-00,BORGNE,G1,680,1,400,800

08-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-12-00,BORGNE,G4,510,1,300,600

08-JUN-2007-13-00,BORGNE,G1,680,1,400,800

08-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-13-00,BORGNE,G4,510,1,300,600

08-JUN-2007-14-00,BORGNE,G1,680,1,400,800

08-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-14-00,BORGNE,G4,510,1,300,600

08-JUN-2007-15-00,BORGNE,G1,680,1,400,800

08-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-15-00,BORGNE,G4,510,1,300,600

08-JUN-2007-16-00,BORGNE,G1,680,1,400,800

08-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-16-00,BORGNE,G4,510,1,300,600

08-JUN-2007-17-00,BORGNE,G1,680,1,400,800

08-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-17-00,BORGNE,G4,510,1,300,600

08-JUN-2007-18-00,BORGNE,G1,680,1,400,800

08-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-18-00,BORGNE,G4,510,1,300,600

08-JUN-2007-19-00,BORGNE,G1,680,1,400,800

08-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-19-00,BORGNE,G4,510,1,300,600

08-JUN-2007-20-00,BORGNE,G1,680,1,400,800

08-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-20-00,BORGNE,G4,510,1,300,600

08-JUN-2007-21-00,BORGNE,G1,680,1,400,800

08-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-21-00,BORGNE,G4,480,1,300,600

08-JUN-2007-22-00,BORGNE,G1,680,1,400,800

08-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-22-00,BORGNE,G4,450,1,300,600

08-JUN-2007-23-00,BORGNE,G1,680,1,400,800

08-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

08-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

08-JUN-2007-23-00,BORGNE,G4,425,1,300,600

09-JUN-2007-00-00,BORGNE,G1,680,1,400,800

09-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-00-00,BORGNE,G4,510,1,300,600

09-JUN-2007-01-00,BORGNE,G1,680,1,400,800

09-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-01-00,BORGNE,G4,510,1,300,600

09-JUN-2007-02-00,BORGNE,G1,680,1,400,800

09-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-02-00,BORGNE,G4,510,1,300,600

09-JUN-2007-03-00,BORGNE,G1,680,1,400,800

09-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-03-00,BORGNE,G4,510,1,300,600

09-JUN-2007-04-00,BORGNE,G1,680,1,400,800

09-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-04-00,BORGNE,G4,510,1,300,600

09-JUN-2007-05-00,BORGNE,G1,680,1,400,800

09-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-05-00,BORGNE,G4,510,1,300,600

09-JUN-2007-06-00,BORGNE,G1,680,1,400,800

09-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-06-00,BORGNE,G4,510,1,300,600

09-JUN-2007-07-00,BORGNE,G1,680,1,400,800

09-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-07-00,BORGNE,G4,510,1,300,600

09-JUN-2007-08-00,BORGNE,G1,680,1,400,800

09-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-08-00,BORGNE,G4,510,1,300,600

09-JUN-2007-09-00,BORGNE,G1,680,1,400,800

09-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-09-00,BORGNE,G4,510,1,300,600

09-JUN-2007-10-00,BORGNE,G1,680,1,400,800

09-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-10-00,BORGNE,G4,510,1,300,600

09-JUN-2007-11-00,BORGNE,G1,680,1,400,800

09-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-11-00,BORGNE,G4,510,1,300,600

09-JUN-2007-11-00,PONTCHATRAIN,G5,210,1,200,400

09-JUN-2007-12-00,BORGNE,G1,680,1,400,800

09-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-12-00,BORGNE,G4,510,1,300,600

09-JUN-2007-12-00,PONTCHATRAIN,G5,225,1,200,400

09-JUN-2007-13-00,BORGNE,G1,680,1,400,800

09-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-13-00,BORGNE,G4,510,1,300,600

09-JUN-2007-13-00,PONTCHATRAIN,G5,240,1,200,400

09-JUN-2007-14-00,BORGNE,G1,680,1,400,800

09-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-14-00,BORGNE,G4,510,1,300,600

09-JUN-2007-14-00,PONTCHATRAIN,G5,245,1,200,400

09-JUN-2007-15-00,BORGNE,G1,680,1,400,800

09-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-15-00,BORGNE,G4,510,1,300,600

09-JUN-2007-15-00,PONTCHATRAIN,G5,240,1,200,400

09-JUN-2007-16-00,BORGNE,G1,680,1,400,800

09-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-16-00,BORGNE,G4,510,1,300,600

09-JUN-2007-16-00,PONTCHATRAIN,G5,225,1,200,400

09-JUN-2007-17-00,BORGNE,G1,680,1,400,800

09-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-17-00,BORGNE,G4,510,1,300,600

09-JUN-2007-17-00,PONTCHATRAIN,G5,210,1,200,400

09-JUN-2007-18-00,BORGNE,G1,680,1,400,800

09-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-18-00,BORGNE,G4,510,1,300,600

09-JUN-2007-19-00,BORGNE,G1,680,1,400,800

09-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-19-00,BORGNE,G4,510,1,300,600

09-JUN-2007-20-00,BORGNE,G1,680,1,400,800

09-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-20-00,BORGNE,G4,510,1,300,600

09-JUN-2007-21-00,BORGNE,G1,680,1,400,800

09-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-21-00,BORGNE,G4,510,1,300,600

09-JUN-2007-22-00,BORGNE,G1,680,1,400,800

09-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-22-00,BORGNE,G4,510,1,300,600

09-JUN-2007-23-00,BORGNE,G1,680,1,400,800

09-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

09-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

09-JUN-2007-23-00,BORGNE,G4,510,1,300,600

10-JUN-2007-00-00,BORGNE,G1,680,1,400,800

10-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-00-00,BORGNE,G4,510,1,300,600

10-JUN-2007-01-00,BORGNE,G1,680,1,400,800

10-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-01-00,BORGNE,G4,510,1,300,600

10-JUN-2007-02-00,BORGNE,G1,680,1,400,800

10-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-02-00,BORGNE,G4,510,1,300,600

10-JUN-2007-03-00,BORGNE,G1,680,1,400,800

10-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-03-00,BORGNE,G4,510,1,300,600

10-JUN-2007-04-00,BORGNE,G1,680,1,400,800

10-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-04-00,BORGNE,G4,510,1,300,600

10-JUN-2007-05-00,BORGNE,G1,680,1,400,800

10-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-05-00,BORGNE,G4,510,1,300,600

10-JUN-2007-06-00,BORGNE,G1,680,1,400,800

10-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-06-00,BORGNE,G4,510,1,300,600

10-JUN-2007-07-00,BORGNE,G1,680,1,400,800

10-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-07-00,BORGNE,G4,510,1,300,600

10-JUN-2007-07-00,PONTCHATRAIN,G5,205,1,200,400

10-JUN-2007-08-00,BORGNE,G1,680,1,400,800

10-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-08-00,BORGNE,G4,510,1,300,600

10-JUN-2007-08-00,PONTCHATRAIN,G5,240,1,200,400

10-JUN-2007-09-00,BORGNE,G1,680,1,400,800

10-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-09-00,BORGNE,G4,510,1,300,600

10-JUN-2007-09-00,PONTCHATRAIN,G5,270,1,200,400

10-JUN-2007-10-00,BORGNE,G1,680,1,400,800

10-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-10-00,BORGNE,G4,510,1,300,600

10-JUN-2007-10-00,PONTCHATRAIN,G5,300,1,200,400

10-JUN-2007-11-00,BORGNE,G1,680,1,400,800

10-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-11-00,BORGNE,G4,510,1,300,600

10-JUN-2007-11-00,PONTCHATRAIN,G5,325,1,200,400

10-JUN-2007-12-00,BORGNE,G1,680,1,400,800

10-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-12-00,BORGNE,G4,510,1,300,600

10-JUN-2007-12-00,PONTCHATRAIN,G5,340,1,200,400

10-JUN-2007-13-00,BORGNE,G1,680,1,400,800

10-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-13-00,BORGNE,G4,510,1,300,600

10-JUN-2007-13-00,PONTCHATRAIN,G5,340,1,200,400

10-JUN-2007-14-00,BORGNE,G1,680,1,400,800

10-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-14-00,BORGNE,G4,510,1,300,600

10-JUN-2007-14-00,PONTCHATRAIN,G5,340,1,200,400

10-JUN-2007-15-00,BORGNE,G1,680,1,400,800

10-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-15-00,BORGNE,G4,510,1,300,600

10-JUN-2007-15-00,PONTCHATRAIN,G5,340,1,200,400

10-JUN-2007-16-00,BORGNE,G1,680,1,400,800

10-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-16-00,BORGNE,G4,510,1,300,600

10-JUN-2007-16-00,PONTCHATRAIN,G5,340,1,200,400

10-JUN-2007-17-00,BORGNE,G1,680,1,400,800

10-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-17-00,BORGNE,G4,510,1,300,600

10-JUN-2007-17-00,PONTCHATRAIN,G5,325,1,200,400

10-JUN-2007-18-00,BORGNE,G1,680,1,400,800

10-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-18-00,BORGNE,G4,510,1,300,600

10-JUN-2007-18-00,PONTCHATRAIN,G5,300,1,200,400

10-JUN-2007-19-00,BORGNE,G1,680,1,400,800

10-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-19-00,BORGNE,G4,510,1,300,600

10-JUN-2007-19-00,PONTCHATRAIN,G5,270,1,200,400

10-JUN-2007-20-00,BORGNE,G1,680,1,400,800

10-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-20-00,BORGNE,G4,510,1,300,600

10-JUN-2007-20-00,PONTCHATRAIN,G5,240,1,200,400

10-JUN-2007-21-00,BORGNE,G1,680,1,400,800

10-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-21-00,BORGNE,G4,510,1,300,600

10-JUN-2007-21-00,PONTCHATRAIN,G5,205,1,200,400

10-JUN-2007-22-00,BORGNE,G1,680,1,400,800

10-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-22-00,BORGNE,G4,510,1,300,600

10-JUN-2007-23-00,BORGNE,G1,680,1,400,800

10-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

10-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

10-JUN-2007-23-00,BORGNE,G4,510,1,300,600

11-JUN-2007-00-00,BORGNE,G1,680,1,400,800

11-JUN-2007-00-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-00-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-00-00,BORGNE,G4,510,1,300,600

11-JUN-2007-00-00,PONTCHATRAIN,G5,235,1,200,400

11-JUN-2007-01-00,BORGNE,G1,680,1,400,800

11-JUN-2007-01-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-01-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-01-00,BORGNE,G4,510,1,300,600

11-JUN-2007-01-00,PONTCHATRAIN,G5,225,1,200,400

11-JUN-2007-02-00,BORGNE,G1,680,1,400,800

11-JUN-2007-02-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-02-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-02-00,BORGNE,G4,510,1,300,600

11-JUN-2007-02-00,PONTCHATRAIN,G5,220,1,200,400

11-JUN-2007-03-00,BORGNE,G1,680,1,400,800

11-JUN-2007-03-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-03-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-03-00,BORGNE,G4,510,1,300,600

11-JUN-2007-03-00,PONTCHATRAIN,G5,225,1,200,400

11-JUN-2007-04-00,BORGNE,G1,680,1,400,800

11-JUN-2007-04-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-04-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-04-00,BORGNE,G4,510,1,300,600

11-JUN-2007-04-00,PONTCHATRAIN,G5,235,1,200,400

11-JUN-2007-05-00,BORGNE,G1,680,1,400,800

11-JUN-2007-05-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-05-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-05-00,BORGNE,G4,510,1,300,600

11-JUN-2007-05-00,PONTCHATRAIN,G5,255,1,200,400

11-JUN-2007-06-00,BORGNE,G1,680,1,400,800

11-JUN-2007-06-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-06-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-06-00,BORGNE,G4,510,1,300,600

11-JUN-2007-06-00,PONTCHATRAIN,G5,280,1,200,400

11-JUN-2007-07-00,BORGNE,G1,680,1,400,800

11-JUN-2007-07-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-07-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-07-00,BORGNE,G4,510,1,300,600

11-JUN-2007-07-00,PONTCHATRAIN,G5,310,1,200,400

11-JUN-2007-08-00,BORGNE,G1,680,1,400,800

11-JUN-2007-08-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-08-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-08-00,BORGNE,G4,510,1,300,600

11-JUN-2007-08-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-09-00,BORGNE,G1,680,1,400,800

11-JUN-2007-09-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-09-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-09-00,BORGNE,G4,510,1,300,600

11-JUN-2007-09-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-10-00,BORGNE,G1,680,1,400,800

11-JUN-2007-10-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-10-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-10-00,BORGNE,G4,510,1,300,600

11-JUN-2007-10-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-11-00,BORGNE,G1,680,1,400,800

11-JUN-2007-11-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-11-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-11-00,BORGNE,G4,510,1,300,600

11-JUN-2007-11-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-12-00,BORGNE,G1,680,1,400,800

11-JUN-2007-12-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-12-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-12-00,BORGNE,G4,510,1,300,600

11-JUN-2007-12-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-13-00,BORGNE,G1,680,1,400,800

11-JUN-2007-13-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-13-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-13-00,BORGNE,G4,510,1,300,600

11-JUN-2007-13-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-14-00,BORGNE,G1,680,1,400,800

11-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-14-00,BORGNE,G4,510,1,300,600

11-JUN-2007-14-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-15-00,BORGNE,G1,680,1,400,800

11-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-15-00,BORGNE,G4,510,1,300,600

11-JUN-2007-15-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-16-00,BORGNE,G1,680,1,400,800

11-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-16-00,BORGNE,G4,510,1,300,600

11-JUN-2007-16-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-17-00,BORGNE,G1,680,1,400,800

11-JUN-2007-17-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-17-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-17-00,BORGNE,G4,510,1,300,600

11-JUN-2007-17-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-18-00,BORGNE,G1,680,1,400,800

11-JUN-2007-18-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-18-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-18-00,BORGNE,G4,510,1,300,600

11-JUN-2007-18-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-19-00,BORGNE,G1,680,1,400,800

11-JUN-2007-19-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-19-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-19-00,BORGNE,G4,510,1,300,600

11-JUN-2007-19-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-20-00,BORGNE,G1,680,1,400,800

11-JUN-2007-20-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-20-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-20-00,BORGNE,G4,510,1,300,600

11-JUN-2007-20-00,PONTCHATRAIN,G5,340,1,200,400

11-JUN-2007-21-00,BORGNE,G1,680,1,400,800

11-JUN-2007-21-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-21-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-21-00,BORGNE,G4,510,1,300,600

11-JUN-2007-21-00,PONTCHATRAIN,G5,310,1,200,400

11-JUN-2007-22-00,BORGNE,G1,680,1,400,800

11-JUN-2007-22-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-22-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-22-00,BORGNE,G4,510,1,300,600

11-JUN-2007-22-00,PONTCHATRAIN,G5,280,1,200,400

11-JUN-2007-23-00,BORGNE,G1,680,1,400,800

11-JUN-2007-23-00,PONTCHATRAIN,G2,1020,1,600,1200

11-JUN-2007-23-00,MAUREPAS,G3,850,1,500,1000

11-JUN-2007-23-00,BORGNE,G4,510,1,300,600

11-JUN-2007-23-00,PONTCHATRAIN,G5,255,1,200,400

12-JUN-2007-14-00,BORGNE,G1,680,1,400,800

12-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

12-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

12-JUN-2007-14-00,BORGNE,G4,510,1,300,600

12-JUN-2007-14-00,PONTCHATRAIN,G5,340,1,200,400

13-JUN-2007-15-00,BORGNE,G1,680,1,400,800

13-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

13-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

13-JUN-2007-15-00,BORGNE,G4,510,1,300,600

13-JUN-2007-15-00,PONTCHATRAIN,G5,340,1,200,400

13-JUN-2007-15-00,MAUREPAS,G6,260,1,250,500

14-JUN-2007-16-00,BORGNE,G1,680,1,400,800

14-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

14-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

14-JUN-2007-16-00,BORGNE,G4,510,1,300,600

14-JUN-2007-16-00,PONTCHATRAIN,G5,340,1,200,400

14-JUN-2007-16-00,MAUREPAS,G6,290,1,250,500

15-JUN-2007-14-00,BORGNE,G1,680,1,400,800

15-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

15-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

15-JUN-2007-14-00,BORGNE,G4,510,1,300,600

15-JUN-2007-14-00,PONTCHATRAIN,G5,340,1,200,400

15-JUN-2007-14-00,MAUREPAS,G6,320,1,250,500

16-JUN-2007-15-00,BORGNE,G1,680,1,400,800

16-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

16-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

16-JUN-2007-15-00,BORGNE,G4,510,1,300,600

16-JUN-2007-15-00,PONTCHATRAIN,G5,340,1,200,400

16-JUN-2007-15-00,MAUREPAS,G6,300,1,250,500

17-JUN-2007-16-00,BORGNE,G1,680,1,400,800

17-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

17-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

17-JUN-2007-16-00,BORGNE,G4,510,1,300,600

17-JUN-2007-16-00,PONTCHATRAIN,G5,340,1,200,400

17-JUN-2007-16-00,MAUREPAS,G6,250,1,250,500

18-JUN-2007-14-00,BORGNE,G1,680,1,400,800

18-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

18-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

18-JUN-2007-14-00,BORGNE,G4,510,1,300,600

18-JUN-2007-14-00,PONTCHATRAIN,G5,340,1,200,400

19-JUN-2007-15-00,BORGNE,G1,680,1,400,800

19-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

19-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

19-JUN-2007-15-00,BORGNE,G4,510,1,300,600

19-JUN-2007-15-00,PONTCHATRAIN,G5,340,1,200,400

20-JUN-2007-16-00,BORGNE,G1,680,1,400,800

20-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

20-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

20-JUN-2007-16-00,BORGNE,G4,510,1,300,600

20-JUN-2007-16-00,PONTCHATRAIN,G5,340,1,200,400

21-JUN-2007-14-00,BORGNE,G1,680,1,400,800

21-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

21-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

21-JUN-2007-14-00,BORGNE,G4,510,1,300,600

21-JUN-2007-14-00,PONTCHATRAIN,G5,245,1,200,400

22-JUN-2007-15-00,BORGNE,G1,680,1,400,800

22-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

22-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

22-JUN-2007-15-00,BORGNE,G4,510,1,300,600

23-JUN-2007-16-00,BORGNE,G1,680,1,400,800

23-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

23-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

23-JUN-2007-16-00,BORGNE,G4,490,1,300,600

24-JUN-2007-14-00,BORGNE,G1,680,1,400,800

24-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

24-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

24-JUN-2007-14-00,BORGNE,G4,385,1,300,600

25-JUN-2007-15-00,BORGNE,G1,680,1,400,800

25-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

25-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

26-JUN-2007-16-00,BORGNE,G1,680,1,400,800

26-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

26-JUN-2007-16-00,MAUREPAS,G3,850,1,500,1000

27-JUN-2007-14-00,BORGNE,G1,680,1,400,800

27-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

27-JUN-2007-14-00,MAUREPAS,G3,850,1,500,1000

28-JUN-2007-15-00,BORGNE,G1,680,1,400,800

28-JUN-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

28-JUN-2007-15-00,MAUREPAS,G3,850,1,500,1000

29-JUN-2007-16-00,BORGNE,G1,680,1,400,800

29-JUN-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

29-JUN-2007-16-00,MAUREPAS,G3,815,1,500,1000

30-JUN-2007-14-00,BORGNE,G1,680,1,400,800

30-JUN-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

30-JUN-2007-14-00,MAUREPAS,G3,820,1,500,1000

01-JUL-2007-15-00,BORGNE,G1,680,1,400,800

01-JUL-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

01-JUL-2007-15-00,MAUREPAS,G3,825,1,500,1000

02-JUL-2007-16-00,BORGNE,G1,680,1,400,800

02-JUL-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

02-JUL-2007-16-00,MAUREPAS,G3,850,1,500,1000

03-JUL-2007-14-00,BORGNE,G1,680,1,400,800

03-JUL-2007-14-00,PONTCHATRAIN,G2,1020,1,600,1200

03-JUL-2007-14-00,MAUREPAS,G3,850,1,500,1000

04-JUL-2007-15-00,BORGNE,G1,680,1,400,800

04-JUL-2007-15-00,PONTCHATRAIN,G2,1020,1,600,1200

04-JUL-2007-15-00,MAUREPAS,G3,850,1,500,1000

05-JUL-2007-16-00,BORGNE,G1,680,1,400,800

05-JUL-2007-16-00,PONTCHATRAIN,G2,1020,1,600,1200

05-JUL-2007-16-00,MAUREPAS,G3,850,1,500,1000

## TSR Reservation Stack Data File Example

The following TSR Stack Data File example matches the projected load provided in the corresponding LF Data File in this appendix.  TSR’s defined under the assumptions in the Overview section of this appendix were selected in random order to re-enforce the provision that the Network Customer may provide TSR’s in any order desired and that the set of TSR’s for any time period may be independent of another time period.

TSR’s are “fully consumed” in the order provided to meet the projected load for the respective time periods.  In this example the cumulative amount of MW’s available for the listed TSR’s exceeds the projected load for that period.  This is acceptable as the AFC process will refine the dispatch of individual TSR’s accordingly.

### Data File Beginning with Record 1:

01-JUN-2007, 0, 12345100

01-JUN-2007, 0, 12345180

01-JUN-2007, 1, 12345020

01-JUN-2007, 1, 12345240

02-JUN-2007, 0, 12345410

02-JUN-2007, 0, 12345060

02-JUN-2007, 1, 12345140

02-JUN-2007, 1, 12345340

03-JUN-2007, 0, 12345130

03-JUN-2007, 0, 12345200

03-JUN-2007, 1, 12345260

03-JUN-2007, 1, 12345060

04-JUN-2007, 0, 12345380

04-JUN-2007, 0, 12345050

04-JUN-2007, 1, 12345260

04-JUN-2007, 1, 12345340

05-JUN-2007, 0, 12345070

05-JUN-2007, 0, 12345350

05-JUN-2007, 1, 12345150

05-JUN-2007, 1, 12345020

06-JUN-2007, 0, 12345210

06-JUN-2007, 0, 12345010

06-JUN-2007, 1, 12345180

06-JUN-2007, 1, 12345130

07-JUN-2007, 0, 12345200

07-JUN-2007, 0, 12345390

07-JUN-2007, 0, 12345180

07-JUN-2007, 1, 12345070

07-JUN-2007, 1, 12345260

07-JUN-2007, 1, 12345300

08-JUN-2007, 0, 12345290

08-JUN-2007, 0, 12345380

08-JUN-2007, 0, 12345390

08-JUN-2007, 1, 12345080

08-JUN-2007, 1, 12345400

08-JUN-2007, 1, 12345180

09-JUN-2007, 0, 12345100

09-JUN-2007, 0, 12345330

09-JUN-2007, 0, 12345020

09-JUN-2007, 1, 12345340

09-JUN-2007, 1, 12345160

09-JUN-2007, 1, 12345270

10-JUN-2007, 0, 12345390

10-JUN-2007, 0, 12345340

10-JUN-2007, 0, 12345150

10-JUN-2007, 1, 12345400

10-JUN-2007, 1, 12345180

10-JUN-2007, 1, 12345140

11-JUN-2007, 0, 12345390

11-JUN-2007, 0, 12345110

11-JUN-2007, 0, 12345010

11-JUN-2007, 1, 12345360

11-JUN-2007, 1, 12345280

11-JUN-2007, 1, 12345090

12-JUN-2007, 1, 12345110

12-JUN-2007, 1, 12345330

12-JUN-2007, 1, 12345020

13-JUN-2007, 1, 12345390

13-JUN-2007, 1, 12345350

13-JUN-2007, 1, 12345020

14-JUN-2007, 1, 12345290

14-JUN-2007, 1, 12345110

14-JUN-2007, 1, 12345170

15-JUN-2007, 1, 12345080

15-JUN-2007, 1, 12345040

15-JUN-2007, 1, 12345330

16-JUN-2007, 1, 12345310

16-JUN-2007, 1, 12345150

16-JUN-2007, 1, 12345090

17-JUN-2007, 1, 12345300

17-JUN-2007, 1, 12345160

17-JUN-2007, 1, 12345250

18-JUN-2007, 1, 12345100

18-JUN-2007, 1, 12345250

18-JUN-2007, 1, 12345110

19-JUN-2007, 1, 12345270

19-JUN-2007, 1, 12345130

19-JUN-2007, 1, 12345380

20-JUN-2007, 1, 12345330

20-JUN-2007, 1, 12345360

20-JUN-2007, 1, 12345230

21-JUN-2007, 1, 12345060

21-JUN-2007, 1, 12345360

21-JUN-2007, 1, 12345280

22-JUN-2007, 1, 12345290

22-JUN-2007, 1, 12345030

22-JUN-2007, 1, 12345220

23-JUN-2007, 1, 12345290

23-JUN-2007, 1, 12345370

23-JUN-2007, 1, 12345060

24-JUN-2007, 1, 12345330

24-JUN-2007, 1, 12345050

25-JUN-2007, 1, 12345370

25-JUN-2007, 1, 12345110

26-JUN-2007, 1, 12345130

26-JUN-2007, 1, 12345070

27-JUN-2007, 1, 12345170

27-JUN-2007, 1, 12345060

28-JUN-2007, 1, 12345270

28-JUN-2007, 1, 12345210

29-JUN-2007, 1, 12345170

29-JUN-2007, 1, 12345100

30-JUN-2007, 1, 12345170

30-JUN-2007, 1, 12345130

01-JUL-2007, 1, 12345030

01-JUL-2007, 1, 12345110

02-JUL-2007, 1, 12345240

02-JUL-2007, 1, 12345120

03-JUL-2007, 1, 12345350

03-JUL-2007, 1, 12345080

04-JUL-2007, 1, 12345130

04-JUL-2007, 1, 12345400

05-JUL-2007, 1, 12345250

05-JUL-2007, 1, 12345290

## TSR Hourly Dispatch Data File Example

The following TSR Hourly Data File example matches the projected load provided in the corresponding LF Data File in this appendix.  For simplicity sake in preparing this example, TSR’s defined under the assumptions in the Overview section of this appendix were selected in sequential order.  While each TSR is defined to reflect 1500MW, a random dispatch amount for each TSR was used to re-enforce the provision that the Network Customer may provide any portion of a TSR and that the dispatch of an individual TSR for any time period may be independent of another time period.

TSR’s are “consumed” in the order provided to meet the projected load for the respective time periods.  In this example the cumulative amount of MW’s available for the listed TSR’s exceeds the projected load for that period.  This is acceptable as the AFC process will refine the dispatch of individual TSR’s accordingly.

(Note: Day 2 through Day 11 may be revealed using “Show/Hide Paragraph” button on Microsoft Word Standard Toolbar.)

### Data File Beginning with Record 1:

01-JUN-2007-00-00,12345010,1050

01-JUN-2007-00-00,12345020,225

01-JUN-2007-00-00,12345030,525

01-JUN-2007-00-00,12345040,525

01-JUN-2007-01-00,12345010,750

01-JUN-2007-01-00,12345020,375

01-JUN-2007-01-00,12345030,1350

01-JUN-2007-02-00,12345010,1275

01-JUN-2007-02-00,12345020,1125

01-JUN-2007-03-00,12345010,825

01-JUN-2007-03-00,12345020,75

01-JUN-2007-03-00,12345030,300

01-JUN-2007-03-00,12345040,1500

01-JUN-2007-04-00,12345010,75

01-JUN-2007-04-00,12345020,1200

01-JUN-2007-04-00,12345030,600

01-JUN-2007-04-00,12345040,450

01-JUN-2007-05-00,12345010,150

01-JUN-2007-05-00,12345020,1425

01-JUN-2007-05-00,12345030,1200

01-JUN-2007-06-00,12345010,525

01-JUN-2007-06-00,12345020,825

01-JUN-2007-06-00,12345030,1275

01-JUN-2007-07-00,12345010,1200

01-JUN-2007-07-00,12345020,525

01-JUN-2007-07-00,12345030,675

01-JUN-2007-08-00,12345010,1275

01-JUN-2007-08-00,12345020,225

01-JUN-2007-08-00,12345030,675

01-JUN-2007-08-00,12345040,600

01-JUN-2007-09-00,12345010,1425

01-JUN-2007-09-00,12345020,900

01-JUN-2007-09-00,12345030,900

01-JUN-2007-10-00,12345010,375

01-JUN-2007-10-00,12345020,900

01-JUN-2007-10-00,12345030,825

01-JUN-2007-10-00,12345040,450

01-JUN-2007-11-00,12345010,1125

01-JUN-2007-11-00,12345020,1425

01-JUN-2007-12-00,12345010,900

01-JUN-2007-12-00,12345020,1425

01-JUN-2007-12-00,12345030,1050

01-JUN-2007-13-00,12345010,225

01-JUN-2007-13-00,12345020,525

01-JUN-2007-13-00,12345030,525

01-JUN-2007-13-00,12345040,750

01-JUN-2007-13-00,12345050,375

01-JUN-2007-13-00,12345060,1350

01-JUN-2007-14-00,12345010,1275

01-JUN-2007-14-00,12345020,1125

01-JUN-2007-14-00,12345030,825

01-JUN-2007-15-00,12345010,75

01-JUN-2007-15-00,12345020,300

01-JUN-2007-15-00,12345030,1500

01-JUN-2007-15-00,12345040,75

01-JUN-2007-15-00,12345050,1200

01-JUN-2007-16-00,12345010,600

01-JUN-2007-16-00,12345020,450

01-JUN-2007-16-00,12345030,150

01-JUN-2007-16-00,12345040,1425

01-JUN-2007-17-00,12345010,1200

01-JUN-2007-17-00,12345020,525

01-JUN-2007-17-00,12345030,825

01-JUN-2007-18-00,12345010,1275

01-JUN-2007-18-00,12345020,1200

01-JUN-2007-19-00,12345010,525

01-JUN-2007-19-00,12345020,675

01-JUN-2007-19-00,12345030,1275

01-JUN-2007-20-00,12345010,225

01-JUN-2007-20-00,12345020,675

01-JUN-2007-20-00,12345030,600

01-JUN-2007-20-00,12345040,1425

01-JUN-2007-21-00,12345010,900

01-JUN-2007-21-00,12345020,900

01-JUN-2007-21-00,12345030,375

01-JUN-2007-21-00,12345040,900

01-JUN-2007-22-00,12345010,825

01-JUN-2007-22-00,12345020,450

01-JUN-2007-22-00,12345030,1125

01-JUN-2007-23-00,12345010,1425

01-JUN-2007-23-00,12345020,900

02-JUN-2007-00-00,12345010,1425

02-JUN-2007-00-00,12345020,1050

02-JUN-2007-01-00,12345010,225

02-JUN-2007-01-00,12345020,525

02-JUN-2007-01-00,12345030,525

02-JUN-2007-01-00,12345040,750

02-JUN-2007-01-00,12345050,375

02-JUN-2007-02-00,12345010,1350

02-JUN-2007-02-00,12345020,1275

02-JUN-2007-03-00,12345010,1125

02-JUN-2007-03-00,12345020,825

02-JUN-2007-03-00,12345030,75

02-JUN-2007-03-00,12345040,300

02-JUN-2007-03-00,12345050,1500

02-JUN-2007-04-00,12345010,75

02-JUN-2007-04-00,12345020,1200

02-JUN-2007-04-00,12345030,600

02-JUN-2007-04-00,12345040,450

02-JUN-2007-04-00,12345050,150

02-JUN-2007-05-00,12345010,1425

02-JUN-2007-05-00,12345020,1200

02-JUN-2007-06-00,12345010,525

02-JUN-2007-06-00,12345020,825

02-JUN-2007-06-00,12345030,1275

02-JUN-2007-07-00,12345010,1200

02-JUN-2007-07-00,12345020,525

02-JUN-2007-07-00,12345030,675

02-JUN-2007-07-00,12345040,1275

02-JUN-2007-08-00,12345010,225

02-JUN-2007-08-00,12345020,675

02-JUN-2007-08-00,12345030,600

02-JUN-2007-08-00,12345040,1425

02-JUN-2007-09-00,12345010,900

02-JUN-2007-09-00,12345020,900

02-JUN-2007-09-00,12345030,375

02-JUN-2007-09-00,12345040,900

02-JUN-2007-10-00,12345010,825

02-JUN-2007-10-00,12345020,450

02-JUN-2007-10-00,12345030,1125

02-JUN-2007-10-00,12345040,1425

02-JUN-2007-11-00,12345010,900

02-JUN-2007-11-00,12345020,1425

02-JUN-2007-11-00,12345030,1050

02-JUN-2007-12-00,12345010,225

02-JUN-2007-12-00,12345020,525

02-JUN-2007-12-00,12345030,525

02-JUN-2007-12-00,12345040,750

02-JUN-2007-12-00,12345050,375

02-JUN-2007-12-00,12345060,1350

02-JUN-2007-13-00,12345010,1275

02-JUN-2007-13-00,12345020,1125

02-JUN-2007-13-00,12345030,825

02-JUN-2007-14-00,12345010,75

02-JUN-2007-14-00,12345020,300

02-JUN-2007-14-00,12345030,1500

02-JUN-2007-14-00,12345040,75

02-JUN-2007-14-00,12345050,1200

02-JUN-2007-15-00,12345010,600

02-JUN-2007-15-00,12345020,450

02-JUN-2007-15-00,12345030,150

02-JUN-2007-15-00,12345040,1425

02-JUN-2007-16-00,12345010,1200

02-JUN-2007-16-00,12345020,525

02-JUN-2007-16-00,12345030,825

02-JUN-2007-16-00,12345040,1275

02-JUN-2007-17-00,12345010,1200

02-JUN-2007-17-00,12345020,525

02-JUN-2007-17-00,12345030,675

02-JUN-2007-17-00,12345040,1275

02-JUN-2007-18-00,12345010,225

02-JUN-2007-18-00,12345020,675

02-JUN-2007-18-00,12345030,600

02-JUN-2007-18-00,12345040,1425

02-JUN-2007-19-00,12345010,900

02-JUN-2007-19-00,12345020,900

02-JUN-2007-19-00,12345030,375

02-JUN-2007-19-00,12345040,900

02-JUN-2007-20-00,12345010,825

02-JUN-2007-20-00,12345020,450

02-JUN-2007-20-00,12345030,1125

02-JUN-2007-20-00,12345040,1425

02-JUN-2007-21-00,12345010,900

02-JUN-2007-21-00,12345020,1425

02-JUN-2007-21-00,12345030,1050

02-JUN-2007-22-00,12345010,225

02-JUN-2007-22-00,12345020,525

02-JUN-2007-22-00,12345030,525

02-JUN-2007-22-00,12345040,750

02-JUN-2007-22-00,12345050,375

02-JUN-2007-23-00,12345010,1350

02-JUN-2007-23-00,12345020,1275

03-JUN-2007-00-00,12345010,1125

03-JUN-2007-00-00,12345020,825

03-JUN-2007-00-00,12345030,75

03-JUN-2007-00-00,12345040,300

03-JUN-2007-00-00,12345050,1500

03-JUN-2007-01-00,12345010,75

03-JUN-2007-01-00,12345020,1200

03-JUN-2007-01-00,12345030,600

03-JUN-2007-01-00,12345040,450

03-JUN-2007-01-00,12345050,150

03-JUN-2007-02-00,12345010,1425

03-JUN-2007-02-00,12345020,1200

03-JUN-2007-03-00,12345010,525

03-JUN-2007-03-00,12345020,825

03-JUN-2007-03-00,12345030,1275

03-JUN-2007-04-00,12345010,1200

03-JUN-2007-04-00,12345020,525

03-JUN-2007-04-00,12345030,675

03-JUN-2007-04-00,12345040,1275

03-JUN-2007-05-00,12345010,225

03-JUN-2007-05-00,12345020,675

03-JUN-2007-05-00,12345030,600

03-JUN-2007-05-00,12345040,1425

03-JUN-2007-06-00,12345010,900

03-JUN-2007-06-00,12345020,900

03-JUN-2007-06-00,12345030,375

03-JUN-2007-06-00,12345040,900

03-JUN-2007-07-00,12345010,825

03-JUN-2007-07-00,12345020,450

03-JUN-2007-07-00,12345030,1125

03-JUN-2007-07-00,12345040,1425

03-JUN-2007-08-00,12345010,900

03-JUN-2007-08-00,12345020,1425

03-JUN-2007-08-00,12345030,1050

03-JUN-2007-09-00,12345010,225

03-JUN-2007-09-00,12345020,525

03-JUN-2007-09-00,12345030,525

03-JUN-2007-09-00,12345040,750

03-JUN-2007-09-00,12345050,375

03-JUN-2007-09-00,12345060,1350

03-JUN-2007-10-00,12345010,1275

03-JUN-2007-10-00,12345020,1125

03-JUN-2007-10-00,12345030,825

03-JUN-2007-11-00,12345010,75

03-JUN-2007-11-00,12345020,300

03-JUN-2007-11-00,12345030,1500

03-JUN-2007-11-00,12345040,75

03-JUN-2007-11-00,12345050,1200

03-JUN-2007-12-00,12345010,600

03-JUN-2007-12-00,12345020,450

03-JUN-2007-12-00,12345030,150

03-JUN-2007-12-00,12345040,1425

03-JUN-2007-13-00,12345010,1200

03-JUN-2007-13-00,12345020,525

03-JUN-2007-13-00,12345030,825

03-JUN-2007-13-00,12345040,1275

03-JUN-2007-14-00,12345010,1200

03-JUN-2007-14-00,12345020,525

03-JUN-2007-14-00,12345030,675

03-JUN-2007-14-00,12345040,1275

03-JUN-2007-15-00,12345010,225

03-JUN-2007-15-00,12345020,675

03-JUN-2007-15-00,12345030,600

03-JUN-2007-15-00,12345040,1425

03-JUN-2007-16-00,12345010,900

03-JUN-2007-16-00,12345020,900

03-JUN-2007-16-00,12345030,375

03-JUN-2007-16-00,12345040,900

03-JUN-2007-17-00,12345010,825

03-JUN-2007-17-00,12345020,450

03-JUN-2007-17-00,12345030,1125

03-JUN-2007-17-00,12345040,1425

03-JUN-2007-18-00,12345010,900

03-JUN-2007-18-00,12345020,1425

03-JUN-2007-18-00,12345030,1050

03-JUN-2007-19-00,12345010,225

03-JUN-2007-19-00,12345020,525

03-JUN-2007-19-00,12345030,525

03-JUN-2007-19-00,12345040,750

03-JUN-2007-19-00,12345050,375

03-JUN-2007-19-00,12345060,1350

03-JUN-2007-20-00,12345010,1275

03-JUN-2007-20-00,12345020,1125

03-JUN-2007-20-00,12345030,825

03-JUN-2007-21-00,12345010,75

03-JUN-2007-21-00,12345020,300

03-JUN-2007-21-00,12345030,1500

03-JUN-2007-21-00,12345040,75

03-JUN-2007-21-00,12345050,1200

03-JUN-2007-22-00,12345010,600

03-JUN-2007-22-00,12345020,450

03-JUN-2007-22-00,12345030,150

03-JUN-2007-22-00,12345040,1425

03-JUN-2007-23-00,12345010,1200

03-JUN-2007-23-00,12345020,525

03-JUN-2007-23-00,12345030,825

04-JUN-2007-00-00,12345010,1275

04-JUN-2007-00-00,12345020,1200

04-JUN-2007-00-00,12345030,525

04-JUN-2007-01-00,12345010,675

04-JUN-2007-01-00,12345020,1275

04-JUN-2007-01-00,12345030,225

04-JUN-2007-01-00,12345040,675

04-JUN-2007-02-00,12345010,600

04-JUN-2007-02-00,12345020,1425

04-JUN-2007-02-00,12345030,900

04-JUN-2007-03-00,12345010,900

04-JUN-2007-03-00,12345020,375

04-JUN-2007-03-00,12345030,900

04-JUN-2007-03-00,12345040,825

04-JUN-2007-04-00,12345010,450

04-JUN-2007-04-00,12345020,1125

04-JUN-2007-04-00,12345030,1425

04-JUN-2007-05-00,12345010,900

04-JUN-2007-05-00,12345020,1425

04-JUN-2007-05-00,12345030,1050

04-JUN-2007-06-00,12345010,225

04-JUN-2007-06-00,12345020,525

04-JUN-2007-06-00,12345030,525

04-JUN-2007-06-00,12345040,750

04-JUN-2007-06-00,12345050,375

04-JUN-2007-06-00,12345060,1350

04-JUN-2007-07-00,12345010,1275

04-JUN-2007-07-00,12345020,1125

04-JUN-2007-07-00,12345030,825

04-JUN-2007-08-00,12345010,75

04-JUN-2007-08-00,12345020,300

04-JUN-2007-08-00,12345030,1500

04-JUN-2007-08-00,12345040,75

04-JUN-2007-08-00,12345050,1200

04-JUN-2007-09-00,12345010,600

04-JUN-2007-09-00,12345020,450

04-JUN-2007-09-00,12345030,150

04-JUN-2007-09-00,12345040,1425

04-JUN-2007-09-00,12345050,1200

04-JUN-2007-10-00,12345010,525

04-JUN-2007-10-00,12345020,825

04-JUN-2007-10-00,12345030,1275

04-JUN-2007-10-00,12345040,1200

04-JUN-2007-11-00,12345010,525

04-JUN-2007-11-00,12345020,675

04-JUN-2007-11-00,12345030,1275

04-JUN-2007-11-00,12345040,225

04-JUN-2007-12-00,12345010,675

04-JUN-2007-12-00,12345020,600

04-JUN-2007-12-00,12345030,1425

04-JUN-2007-13-00,12345010,900

04-JUN-2007-13-00,12345020,900

04-JUN-2007-13-00,12345030,375

04-JUN-2007-13-00,12345040,900

04-JUN-2007-14-00,12345010,825

04-JUN-2007-14-00,12345020,450

04-JUN-2007-14-00,12345030,1125

04-JUN-2007-14-00,12345040,1425

04-JUN-2007-15-00,12345010,900

04-JUN-2007-15-00,12345020,1425

04-JUN-2007-15-00,12345030,1050

04-JUN-2007-16-00,12345010,225

04-JUN-2007-16-00,12345020,525

04-JUN-2007-16-00,12345030,525

04-JUN-2007-16-00,12345040,750

04-JUN-2007-16-00,12345050,375

04-JUN-2007-16-00,12345060,1350

04-JUN-2007-17-00,12345010,1275

04-JUN-2007-17-00,12345020,1125

04-JUN-2007-17-00,12345030,825

04-JUN-2007-18-00,12345010,75

04-JUN-2007-18-00,12345020,300

04-JUN-2007-18-00,12345030,1500

04-JUN-2007-18-00,12345040,75

04-JUN-2007-18-00,12345050,1200

04-JUN-2007-19-00,12345010,600

04-JUN-2007-19-00,12345020,450

04-JUN-2007-19-00,12345030,150

04-JUN-2007-19-00,12345040,1425

04-JUN-2007-19-00,12345050,1200

04-JUN-2007-20-00,12345010,525

04-JUN-2007-20-00,12345020,825

04-JUN-2007-20-00,12345030,1275

04-JUN-2007-21-00,12345010,1200

04-JUN-2007-21-00,12345020,525

04-JUN-2007-21-00,12345030,675

04-JUN-2007-21-00,12345040,1275

04-JUN-2007-22-00,12345010,225

04-JUN-2007-22-00,12345020,675

04-JUN-2007-22-00,12345030,600

04-JUN-2007-22-00,12345040,1425

04-JUN-2007-23-00,12345010,900

04-JUN-2007-23-00,12345020,900

04-JUN-2007-23-00,12345030,375

04-JUN-2007-23-00,12345040,900

05-JUN-2007-00-00,12345010,825

05-JUN-2007-00-00,12345020,450

05-JUN-2007-00-00,12345030,1125

05-JUN-2007-00-00,12345040,1425

05-JUN-2007-01-00,12345010,900

05-JUN-2007-01-00,12345020,1425

05-JUN-2007-01-00,12345030,1050

05-JUN-2007-02-00,12345010,225

05-JUN-2007-02-00,12345020,525

05-JUN-2007-02-00,12345030,525

05-JUN-2007-02-00,12345040,750

05-JUN-2007-02-00,12345050,375

05-JUN-2007-02-00,12345060,1350

05-JUN-2007-03-00,12345010,1275

05-JUN-2007-03-00,12345020,1125

05-JUN-2007-03-00,12345030,825

05-JUN-2007-04-00,12345010,75

05-JUN-2007-04-00,12345020,300

05-JUN-2007-04-00,12345030,1500

05-JUN-2007-04-00,12345040,75

05-JUN-2007-04-00,12345050,1200

05-JUN-2007-05-00,12345010,600

05-JUN-2007-05-00,12345020,450

05-JUN-2007-05-00,12345030,150

05-JUN-2007-05-00,12345040,1425

05-JUN-2007-06-00,12345010,1200

05-JUN-2007-06-00,12345020,525

05-JUN-2007-06-00,12345030,825

05-JUN-2007-06-00,12345040,1275

05-JUN-2007-07-00,12345010,1200

05-JUN-2007-07-00,12345020,525

05-JUN-2007-07-00,12345030,675

05-JUN-2007-07-00,12345040,1275

05-JUN-2007-08-00,12345010,225

05-JUN-2007-08-00,12345020,675

05-JUN-2007-08-00,12345030,600

05-JUN-2007-08-00,12345040,1425

05-JUN-2007-09-00,12345010,900

05-JUN-2007-09-00,12345020,900

05-JUN-2007-09-00,12345030,375

05-JUN-2007-09-00,12345040,900

05-JUN-2007-10-00,12345010,825

05-JUN-2007-10-00,12345020,450

05-JUN-2007-10-00,12345030,1125

05-JUN-2007-10-00,12345040,1425

05-JUN-2007-11-00,12345010,900

05-JUN-2007-11-00,12345020,1425

05-JUN-2007-11-00,12345030,1050

05-JUN-2007-12-00,12345010,225

05-JUN-2007-12-00,12345020,525

05-JUN-2007-12-00,12345030,525

05-JUN-2007-12-00,12345040,750

05-JUN-2007-12-00,12345050,375

05-JUN-2007-12-00,12345060,1350

05-JUN-2007-13-00,12345010,1275

05-JUN-2007-13-00,12345020,1125

05-JUN-2007-13-00,12345030,825

05-JUN-2007-14-00,12345010,75

05-JUN-2007-14-00,12345020,300

05-JUN-2007-14-00,12345030,1500

05-JUN-2007-14-00,12345040,75

05-JUN-2007-14-00,12345050,1200

05-JUN-2007-15-00,12345010,600

05-JUN-2007-15-00,12345020,450

05-JUN-2007-15-00,12345030,150

05-JUN-2007-15-00,12345040,1425

05-JUN-2007-15-00,12345050,1200

05-JUN-2007-16-00,12345010,525

05-JUN-2007-16-00,12345020,825

05-JUN-2007-16-00,12345030,1275

05-JUN-2007-16-00,12345040,1200

05-JUN-2007-17-00,12345010,525

05-JUN-2007-17-00,12345020,675

05-JUN-2007-17-00,12345030,1275

05-JUN-2007-17-00,12345040,225

05-JUN-2007-17-00,12345050,675

05-JUN-2007-18-00,12345010,600

05-JUN-2007-18-00,12345020,1425

05-JUN-2007-18-00,12345030,900

05-JUN-2007-19-00,12345010,900

05-JUN-2007-19-00,12345020,375

05-JUN-2007-19-00,12345030,900

05-JUN-2007-19-00,12345040,825

05-JUN-2007-20-00,12345010,450

05-JUN-2007-20-00,12345020,1125

05-JUN-2007-20-00,12345030,1425

05-JUN-2007-21-00,12345010,900

05-JUN-2007-21-00,12345020,1425

05-JUN-2007-21-00,12345030,1050

05-JUN-2007-22-00,12345010,225

05-JUN-2007-22-00,12345020,525

05-JUN-2007-22-00,12345030,525

05-JUN-2007-22-00,12345040,750

05-JUN-2007-22-00,12345050,375

05-JUN-2007-22-00,12345060,1350

05-JUN-2007-23-00,12345010,1275

05-JUN-2007-23-00,12345020,1125

05-JUN-2007-23-00,12345030,825

06-JUN-2007-00-00,12345010,75

06-JUN-2007-00-00,12345020,300

06-JUN-2007-00-00,12345030,1500

06-JUN-2007-00-00,12345040,75

06-JUN-2007-00-00,12345050,1200

06-JUN-2007-01-00,12345010,600

06-JUN-2007-01-00,12345020,450

06-JUN-2007-01-00,12345030,150

06-JUN-2007-01-00,12345040,1425

06-JUN-2007-01-00,12345050,1200

06-JUN-2007-02-00,12345010,525

06-JUN-2007-02-00,12345020,825

06-JUN-2007-02-00,12345030,1275

06-JUN-2007-02-00,12345040,1200

06-JUN-2007-03-00,12345010,525

06-JUN-2007-03-00,12345020,675

06-JUN-2007-03-00,12345030,1275

06-JUN-2007-03-00,12345040,225

06-JUN-2007-04-00,12345010,675

06-JUN-2007-04-00,12345020,600

06-JUN-2007-04-00,12345030,1425

06-JUN-2007-04-00,12345040,900

06-JUN-2007-05-00,12345010,900

06-JUN-2007-05-00,12345020,375

06-JUN-2007-05-00,12345030,900

06-JUN-2007-05-00,12345040,825

06-JUN-2007-06-00,12345010,450

06-JUN-2007-06-00,12345020,1125

06-JUN-2007-06-00,12345030,1425

06-JUN-2007-07-00,12345010,900

06-JUN-2007-07-00,12345020,1425

06-JUN-2007-07-00,12345030,1050

06-JUN-2007-08-00,12345010,225

06-JUN-2007-08-00,12345020,525

06-JUN-2007-08-00,12345030,525

06-JUN-2007-08-00,12345040,750

06-JUN-2007-08-00,12345050,375

06-JUN-2007-08-00,12345060,1350

06-JUN-2007-09-00,12345010,1275

06-JUN-2007-09-00,12345020,1125

06-JUN-2007-09-00,12345030,825

06-JUN-2007-10-00,12345010,75

06-JUN-2007-10-00,12345020,300

06-JUN-2007-10-00,12345030,1500

06-JUN-2007-10-00,12345040,75

06-JUN-2007-10-00,12345050,1200

06-JUN-2007-11-00,12345010,600

06-JUN-2007-11-00,12345020,450

06-JUN-2007-11-00,12345030,150

06-JUN-2007-11-00,12345040,1425

06-JUN-2007-11-00,12345050,1200

06-JUN-2007-12-00,12345010,525

06-JUN-2007-12-00,12345020,825

06-JUN-2007-12-00,12345030,1275

06-JUN-2007-12-00,12345040,1200

06-JUN-2007-13-00,12345010,525

06-JUN-2007-13-00,12345020,675

06-JUN-2007-13-00,12345030,1275

06-JUN-2007-13-00,12345040,225

06-JUN-2007-13-00,12345050,675

06-JUN-2007-14-00,12345010,600

06-JUN-2007-14-00,12345020,1425

06-JUN-2007-14-00,12345030,900

06-JUN-2007-14-00,12345040,900

06-JUN-2007-15-00,12345010,375

06-JUN-2007-15-00,12345020,900

06-JUN-2007-15-00,12345030,825

06-JUN-2007-15-00,12345040,450

06-JUN-2007-15-00,12345050,1125

06-JUN-2007-16-00,12345010,1425

06-JUN-2007-16-00,12345020,900

06-JUN-2007-16-00,12345030,1425

06-JUN-2007-17-00,12345010,1050

06-JUN-2007-17-00,12345020,225

06-JUN-2007-17-00,12345030,525

06-JUN-2007-17-00,12345040,525

06-JUN-2007-17-00,12345050,750

06-JUN-2007-18-00,12345010,375

06-JUN-2007-18-00,12345020,1350

06-JUN-2007-18-00,12345030,1275

06-JUN-2007-19-00,12345010,1125

06-JUN-2007-19-00,12345020,825

06-JUN-2007-19-00,12345030,75

06-JUN-2007-19-00,12345040,300

06-JUN-2007-19-00,12345050,1500

06-JUN-2007-20-00,12345010,75

06-JUN-2007-20-00,12345020,1200

06-JUN-2007-20-00,12345030,600

06-JUN-2007-20-00,12345040,450

06-JUN-2007-20-00,12345050,150

06-JUN-2007-20-00,12345060,1425

06-JUN-2007-21-00,12345010,1200

06-JUN-2007-21-00,12345020,525

06-JUN-2007-21-00,12345030,825

06-JUN-2007-21-00,12345040,1275

06-JUN-2007-22-00,12345010,1200

06-JUN-2007-22-00,12345020,525

06-JUN-2007-22-00,12345030,675

06-JUN-2007-22-00,12345040,1275

06-JUN-2007-23-00,12345010,225

06-JUN-2007-23-00,12345020,675

06-JUN-2007-23-00,12345030,600

06-JUN-2007-23-00,12345040,1425

07-JUN-2007-00-00,12345010,900

07-JUN-2007-00-00,12345020,900

07-JUN-2007-00-00,12345030,375

07-JUN-2007-00-00,12345040,900

07-JUN-2007-01-00,12345010,825

07-JUN-2007-01-00,12345020,450

07-JUN-2007-01-00,12345030,1125

07-JUN-2007-01-00,12345040,1425

07-JUN-2007-02-00,12345010,900

07-JUN-2007-02-00,12345020,1425

07-JUN-2007-02-00,12345030,1050

07-JUN-2007-03-00,12345010,225

07-JUN-2007-03-00,12345020,525

07-JUN-2007-03-00,12345030,525

07-JUN-2007-03-00,12345040,750

07-JUN-2007-03-00,12345050,375

07-JUN-2007-03-00,12345060,1350

07-JUN-2007-04-00,12345010,1275

07-JUN-2007-04-00,12345020,1125

07-JUN-2007-04-00,12345030,825

07-JUN-2007-05-00,12345010,75

07-JUN-2007-05-00,12345020,300

07-JUN-2007-05-00,12345030,1500

07-JUN-2007-05-00,12345040,75

07-JUN-2007-05-00,12345050,1200

07-JUN-2007-06-00,12345010,600

07-JUN-2007-06-00,12345020,450

07-JUN-2007-06-00,12345030,150

07-JUN-2007-06-00,12345040,1425

07-JUN-2007-06-00,12345050,1200

07-JUN-2007-07-00,12345010,525

07-JUN-2007-07-00,12345020,825

07-JUN-2007-07-00,12345030,1275

07-JUN-2007-07-00,12345040,1200

07-JUN-2007-08-00,12345010,525

07-JUN-2007-08-00,12345020,675

07-JUN-2007-08-00,12345030,1275

07-JUN-2007-08-00,12345040,225

07-JUN-2007-08-00,12345050,675

07-JUN-2007-09-00,12345010,600

07-JUN-2007-09-00,12345020,1425

07-JUN-2007-09-00,12345030,900

07-JUN-2007-09-00,12345040,900

07-JUN-2007-10-00,12345010,375

07-JUN-2007-10-00,12345020,900

07-JUN-2007-10-00,12345030,825

07-JUN-2007-10-00,12345040,450

07-JUN-2007-10-00,12345050,1125

07-JUN-2007-11-00,12345010,1425

07-JUN-2007-11-00,12345020,900

07-JUN-2007-11-00,12345030,1425

07-JUN-2007-12-00,12345010,1050

07-JUN-2007-12-00,12345020,225

07-JUN-2007-12-00,12345030,525

07-JUN-2007-12-00,12345040,525

07-JUN-2007-12-00,12345050,750

07-JUN-2007-13-00,12345010,375

07-JUN-2007-13-00,12345020,1350

07-JUN-2007-13-00,12345030,1275

07-JUN-2007-13-00,12345040,1125

07-JUN-2007-14-00,12345010,825

07-JUN-2007-14-00,12345020,75

07-JUN-2007-14-00,12345030,300

07-JUN-2007-14-00,12345040,1500

07-JUN-2007-14-00,12345050,75

07-JUN-2007-14-00,12345060,1200

07-JUN-2007-15-00,12345010,600

07-JUN-2007-15-00,12345020,450

07-JUN-2007-15-00,12345030,150

07-JUN-2007-15-00,12345040,1425

07-JUN-2007-15-00,12345050,1200

07-JUN-2007-16-00,12345010,525

07-JUN-2007-16-00,12345020,825

07-JUN-2007-16-00,12345030,1275

07-JUN-2007-16-00,12345040,1200

07-JUN-2007-17-00,12345010,525

07-JUN-2007-17-00,12345020,675

07-JUN-2007-17-00,12345030,1275

07-JUN-2007-17-00,12345040,225

07-JUN-2007-17-00,12345050,675

07-JUN-2007-18-00,12345010,600

07-JUN-2007-18-00,12345020,1425

07-JUN-2007-18-00,12345030,900

07-JUN-2007-18-00,12345040,900

07-JUN-2007-19-00,12345010,375

07-JUN-2007-19-00,12345020,900

07-JUN-2007-19-00,12345030,825

07-JUN-2007-19-00,12345040,450

07-JUN-2007-19-00,12345050,1125

07-JUN-2007-20-00,12345010,1425

07-JUN-2007-20-00,12345020,900

07-JUN-2007-20-00,12345030,1425

07-JUN-2007-21-00,12345010,1050

07-JUN-2007-21-00,12345020,225

07-JUN-2007-21-00,12345030,525

07-JUN-2007-21-00,12345040,525

07-JUN-2007-21-00,12345050,750

07-JUN-2007-22-00,12345010,375

07-JUN-2007-22-00,12345020,1350

07-JUN-2007-22-00,12345030,1275

07-JUN-2007-23-00,12345010,1125

07-JUN-2007-23-00,12345020,825

07-JUN-2007-23-00,12345030,75

07-JUN-2007-23-00,12345040,300

07-JUN-2007-23-00,12345050,1500

08-JUN-2007-00-00,12345010,75

08-JUN-2007-00-00,12345020,1200

08-JUN-2007-00-00,12345030,600

08-JUN-2007-00-00,12345040,450

08-JUN-2007-00-00,12345050,150

08-JUN-2007-00-00,12345060,1425

08-JUN-2007-01-00,12345010,1200

08-JUN-2007-01-00,12345020,525

08-JUN-2007-01-00,12345030,825

08-JUN-2007-01-00,12345040,1275

08-JUN-2007-02-00,12345010,1200

08-JUN-2007-02-00,12345020,525

08-JUN-2007-02-00,12345030,675

08-JUN-2007-02-00,12345040,1275

08-JUN-2007-03-00,12345010,225

08-JUN-2007-03-00,12345020,675

08-JUN-2007-03-00,12345030,600

08-JUN-2007-03-00,12345040,1425

08-JUN-2007-03-00,12345050,900

08-JUN-2007-04-00,12345010,900

08-JUN-2007-04-00,12345020,375

08-JUN-2007-04-00,12345030,900

08-JUN-2007-04-00,12345040,825

08-JUN-2007-05-00,12345010,450

08-JUN-2007-05-00,12345020,1125

08-JUN-2007-05-00,12345030,1425

08-JUN-2007-06-00,12345010,900

08-JUN-2007-06-00,12345020,1425

08-JUN-2007-06-00,12345030,1050

08-JUN-2007-07-00,12345010,225

08-JUN-2007-07-00,12345020,525

08-JUN-2007-07-00,12345030,525

08-JUN-2007-07-00,12345040,750

08-JUN-2007-07-00,12345050,375

08-JUN-2007-07-00,12345060,1350

08-JUN-2007-08-00,12345010,1275

08-JUN-2007-08-00,12345020,1125

08-JUN-2007-08-00,12345030,825

08-JUN-2007-09-00,12345010,75

08-JUN-2007-09-00,12345020,300

08-JUN-2007-09-00,12345030,1500

08-JUN-2007-09-00,12345040,75

08-JUN-2007-09-00,12345050,1200

08-JUN-2007-10-00,12345010,600

08-JUN-2007-10-00,12345020,450

08-JUN-2007-10-00,12345030,150

08-JUN-2007-10-00,12345040,1425

08-JUN-2007-10-00,12345050,1200

08-JUN-2007-11-00,12345010,525

08-JUN-2007-11-00,12345020,825

08-JUN-2007-11-00,12345030,1275

08-JUN-2007-11-00,12345040,1200

08-JUN-2007-12-00,12345010,525

08-JUN-2007-12-00,12345020,675

08-JUN-2007-12-00,12345030,1275

08-JUN-2007-12-00,12345040,225

08-JUN-2007-12-00,12345050,675

08-JUN-2007-13-00,12345010,600

08-JUN-2007-13-00,12345020,1425

08-JUN-2007-13-00,12345030,900

08-JUN-2007-13-00,12345040,900

08-JUN-2007-14-00,12345010,375

08-JUN-2007-14-00,12345020,900

08-JUN-2007-14-00,12345030,825

08-JUN-2007-14-00,12345040,450

08-JUN-2007-14-00,12345050,1125

08-JUN-2007-15-00,12345010,1425

08-JUN-2007-15-00,12345020,900

08-JUN-2007-15-00,12345030,1425

08-JUN-2007-16-00,12345010,1050

08-JUN-2007-16-00,12345020,225

08-JUN-2007-16-00,12345030,525

08-JUN-2007-16-00,12345040,525

08-JUN-2007-16-00,12345050,750

08-JUN-2007-16-00,12345060,375

08-JUN-2007-17-00,12345010,1350

08-JUN-2007-17-00,12345020,1275

08-JUN-2007-17-00,12345030,1125

08-JUN-2007-18-00,12345010,825

08-JUN-2007-18-00,12345020,75

08-JUN-2007-18-00,12345030,300

08-JUN-2007-18-00,12345040,1500

08-JUN-2007-18-00,12345050,75

08-JUN-2007-18-00,12345060,1200

08-JUN-2007-19-00,12345010,600

08-JUN-2007-19-00,12345020,450

08-JUN-2007-19-00,12345030,150

08-JUN-2007-19-00,12345040,1425

08-JUN-2007-19-00,12345050,1200

08-JUN-2007-20-00,12345010,525

08-JUN-2007-20-00,12345020,825

08-JUN-2007-20-00,12345030,1275

08-JUN-2007-20-00,12345040,1200

08-JUN-2007-21-00,12345010,525

08-JUN-2007-21-00,12345020,675

08-JUN-2007-21-00,12345030,1275

08-JUN-2007-21-00,12345040,225

08-JUN-2007-21-00,12345050,675

08-JUN-2007-22-00,12345010,600

08-JUN-2007-22-00,12345020,1425

08-JUN-2007-22-00,12345030,900

08-JUN-2007-22-00,12345040,900

08-JUN-2007-23-00,12345010,375

08-JUN-2007-23-00,12345020,900

08-JUN-2007-23-00,12345030,825

08-JUN-2007-23-00,12345040,450

08-JUN-2007-23-00,12345050,1125

09-JUN-2007-00-00,12345010,1425

09-JUN-2007-00-00,12345020,900

09-JUN-2007-00-00,12345030,1425

09-JUN-2007-01-00,12345010,1050

09-JUN-2007-01-00,12345020,225

09-JUN-2007-01-00,12345030,525

09-JUN-2007-01-00,12345040,525

09-JUN-2007-01-00,12345050,750

09-JUN-2007-02-00,12345010,375

09-JUN-2007-02-00,12345020,1350

09-JUN-2007-02-00,12345030,1275

09-JUN-2007-02-00,12345040,1125

09-JUN-2007-03-00,12345010,825

09-JUN-2007-03-00,12345020,75

09-JUN-2007-03-00,12345030,300

09-JUN-2007-03-00,12345040,1500

09-JUN-2007-03-00,12345050,75

09-JUN-2007-03-00,12345060,1200

09-JUN-2007-04-00,12345010,600

09-JUN-2007-04-00,12345020,450

09-JUN-2007-04-00,12345030,150

09-JUN-2007-04-00,12345040,1425

09-JUN-2007-04-00,12345050,1200

09-JUN-2007-05-00,12345010,525

09-JUN-2007-05-00,12345020,825

09-JUN-2007-05-00,12345030,1275

09-JUN-2007-05-00,12345040,1200

09-JUN-2007-06-00,12345010,525

09-JUN-2007-06-00,12345020,675

09-JUN-2007-06-00,12345030,1275

09-JUN-2007-06-00,12345040,225

09-JUN-2007-06-00,12345050,675

09-JUN-2007-07-00,12345010,600

09-JUN-2007-07-00,12345020,1425

09-JUN-2007-07-00,12345030,900

09-JUN-2007-07-00,12345040,900

09-JUN-2007-08-00,12345010,375

09-JUN-2007-08-00,12345020,900

09-JUN-2007-08-00,12345030,825

09-JUN-2007-08-00,12345040,450

09-JUN-2007-08-00,12345050,1125

09-JUN-2007-09-00,12345010,1425

09-JUN-2007-09-00,12345020,900

09-JUN-2007-09-00,12345030,1425

09-JUN-2007-10-00,12345010,1050

09-JUN-2007-10-00,12345020,225

09-JUN-2007-10-00,12345030,525

09-JUN-2007-10-00,12345040,525

09-JUN-2007-10-00,12345050,750

09-JUN-2007-10-00,12345060,375

09-JUN-2007-11-00,12345010,1350

09-JUN-2007-11-00,12345020,1275

09-JUN-2007-11-00,12345030,1125

09-JUN-2007-12-00,12345010,825

09-JUN-2007-12-00,12345020,75

09-JUN-2007-12-00,12345030,300

09-JUN-2007-12-00,12345040,1500

09-JUN-2007-12-00,12345050,75

09-JUN-2007-12-00,12345060,1200

09-JUN-2007-13-00,12345010,600

09-JUN-2007-13-00,12345020,450

09-JUN-2007-13-00,12345030,150

09-JUN-2007-13-00,12345040,1425

09-JUN-2007-13-00,12345050,1200

09-JUN-2007-14-00,12345010,525

09-JUN-2007-14-00,12345020,825

09-JUN-2007-14-00,12345030,1275

09-JUN-2007-14-00,12345040,1200

09-JUN-2007-15-00,12345010,525

09-JUN-2007-15-00,12345020,675

09-JUN-2007-15-00,12345030,1275

09-JUN-2007-15-00,12345040,225

09-JUN-2007-15-00,12345050,675

09-JUN-2007-16-00,12345010,600

09-JUN-2007-16-00,12345020,1425

09-JUN-2007-16-00,12345030,900

09-JUN-2007-16-00,12345040,900

09-JUN-2007-17-00,12345010,375

09-JUN-2007-17-00,12345020,900

09-JUN-2007-17-00,12345030,825

09-JUN-2007-17-00,12345040,450

09-JUN-2007-17-00,12345050,1125

09-JUN-2007-18-00,12345010,1425

09-JUN-2007-18-00,12345020,900

09-JUN-2007-18-00,12345030,1425

09-JUN-2007-19-00,12345010,1050

09-JUN-2007-19-00,12345020,225

09-JUN-2007-19-00,12345030,525

09-JUN-2007-19-00,12345040,525

09-JUN-2007-19-00,12345050,750

09-JUN-2007-19-00,12345060,375

09-JUN-2007-20-00,12345010,1350

09-JUN-2007-20-00,12345020,1275

09-JUN-2007-20-00,12345030,1125

09-JUN-2007-21-00,12345010,825

09-JUN-2007-21-00,12345020,75

09-JUN-2007-21-00,12345030,300

09-JUN-2007-21-00,12345040,1500

09-JUN-2007-21-00,12345050,75

09-JUN-2007-21-00,12345060,1200

09-JUN-2007-22-00,12345010,600

09-JUN-2007-22-00,12345020,450

09-JUN-2007-22-00,12345030,150

09-JUN-2007-22-00,12345040,1425

09-JUN-2007-22-00,12345050,1200

09-JUN-2007-23-00,12345010,525

09-JUN-2007-23-00,12345020,825

09-JUN-2007-23-00,12345030,1275

09-JUN-2007-23-00,12345040,1200

10-JUN-2007-00-00,12345010,525

10-JUN-2007-00-00,12345020,675

10-JUN-2007-00-00,12345030,1275

10-JUN-2007-00-00,12345040,225

10-JUN-2007-00-00,12345050,675

10-JUN-2007-01-00,12345010,600

10-JUN-2007-01-00,12345020,1425

10-JUN-2007-01-00,12345030,900

10-JUN-2007-01-00,12345040,900

10-JUN-2007-02-00,12345010,375

10-JUN-2007-02-00,12345020,900

10-JUN-2007-02-00,12345030,825

10-JUN-2007-02-00,12345040,450

10-JUN-2007-02-00,12345050,1125

10-JUN-2007-03-00,12345010,1425

10-JUN-2007-03-00,12345020,900

10-JUN-2007-03-00,12345030,1425

10-JUN-2007-04-00,12345010,1050

10-JUN-2007-04-00,12345020,225

10-JUN-2007-04-00,12345030,525

10-JUN-2007-04-00,12345040,525

10-JUN-2007-04-00,12345050,750

10-JUN-2007-04-00,12345060,375

10-JUN-2007-05-00,12345010,1350

10-JUN-2007-05-00,12345020,1275

10-JUN-2007-05-00,12345030,1125

10-JUN-2007-06-00,12345010,825

10-JUN-2007-06-00,12345020,75

10-JUN-2007-06-00,12345030,300

10-JUN-2007-06-00,12345040,1500

10-JUN-2007-06-00,12345050,75

10-JUN-2007-06-00,12345060,1200

10-JUN-2007-07-00,12345010,600

10-JUN-2007-07-00,12345020,450

10-JUN-2007-07-00,12345030,150

10-JUN-2007-07-00,12345040,1425

10-JUN-2007-07-00,12345050,1200

10-JUN-2007-08-00,12345010,525

10-JUN-2007-08-00,12345020,825

10-JUN-2007-08-00,12345030,1275

10-JUN-2007-08-00,12345040,1200

10-JUN-2007-09-00,12345010,525

10-JUN-2007-09-00,12345020,675

10-JUN-2007-09-00,12345030,1275

10-JUN-2007-09-00,12345040,225

10-JUN-2007-09-00,12345050,675

10-JUN-2007-10-00,12345010,600

10-JUN-2007-10-00,12345020,1425

10-JUN-2007-10-00,12345030,900

10-JUN-2007-10-00,12345040,900

10-JUN-2007-11-00,12345010,375

10-JUN-2007-11-00,12345020,900

10-JUN-2007-11-00,12345030,825

10-JUN-2007-11-00,12345040,450

10-JUN-2007-11-00,12345050,1125

10-JUN-2007-12-00,12345010,1425

10-JUN-2007-12-00,12345020,900

10-JUN-2007-12-00,12345030,1425

10-JUN-2007-13-00,12345010,1050

10-JUN-2007-13-00,12345020,225

10-JUN-2007-13-00,12345030,525

10-JUN-2007-13-00,12345040,525

10-JUN-2007-13-00,12345050,750

10-JUN-2007-13-00,12345060,375

10-JUN-2007-14-00,12345010,1350

10-JUN-2007-14-00,12345020,1275

10-JUN-2007-14-00,12345030,1125

10-JUN-2007-15-00,12345010,825

10-JUN-2007-15-00,12345020,75

10-JUN-2007-15-00,12345030,300

10-JUN-2007-15-00,12345040,1500

10-JUN-2007-15-00,12345050,75

10-JUN-2007-15-00,12345060,1200

10-JUN-2007-16-00,12345010,600

10-JUN-2007-16-00,12345020,450

10-JUN-2007-16-00,12345030,150

10-JUN-2007-16-00,12345040,1425

10-JUN-2007-16-00,12345050,1200

10-JUN-2007-17-00,12345010,525

10-JUN-2007-17-00,12345020,825

10-JUN-2007-17-00,12345030,1275

10-JUN-2007-17-00,12345040,1200

10-JUN-2007-18-00,12345010,525

10-JUN-2007-18-00,12345020,675

10-JUN-2007-18-00,12345030,1275

10-JUN-2007-18-00,12345040,225

10-JUN-2007-18-00,12345050,675

10-JUN-2007-19-00,12345010,600

10-JUN-2007-19-00,12345020,1425

10-JUN-2007-19-00,12345030,900

10-JUN-2007-19-00,12345040,900

10-JUN-2007-20-00,12345010,375

10-JUN-2007-20-00,12345020,900

10-JUN-2007-20-00,12345030,825

10-JUN-2007-20-00,12345040,450

10-JUN-2007-20-00,12345050,1125

10-JUN-2007-21-00,12345010,1425

10-JUN-2007-21-00,12345020,900

10-JUN-2007-21-00,12345030,1425

10-JUN-2007-22-00,12345010,1050

10-JUN-2007-22-00,12345020,225

10-JUN-2007-22-00,12345030,525

10-JUN-2007-22-00,12345040,525

10-JUN-2007-22-00,12345050,750

10-JUN-2007-22-00,12345060,375

10-JUN-2007-23-00,12345010,1350

10-JUN-2007-23-00,12345020,1275

10-JUN-2007-23-00,12345030,1125

11-JUN-2007-00-00,12345010,825

11-JUN-2007-00-00,12345020,75

11-JUN-2007-00-00,12345030,300

11-JUN-2007-00-00,12345040,1500

11-JUN-2007-00-00,12345050,75

11-JUN-2007-00-00,12345060,1200

11-JUN-2007-01-00,12345010,600

11-JUN-2007-01-00,12345020,450

11-JUN-2007-01-00,12345030,150

11-JUN-2007-01-00,12345040,1425

11-JUN-2007-01-00,12345050,1200

11-JUN-2007-02-00,12345010,525

11-JUN-2007-02-00,12345020,825

11-JUN-2007-02-00,12345030,1275

11-JUN-2007-02-00,12345040,1200

11-JUN-2007-03-00,12345010,525

11-JUN-2007-03-00,12345020,675

11-JUN-2007-03-00,12345030,1275

11-JUN-2007-03-00,12345040,225

11-JUN-2007-03-00,12345050,675

11-JUN-2007-04-00,12345010,600

11-JUN-2007-04-00,12345020,1425

11-JUN-2007-04-00,12345030,900

11-JUN-2007-04-00,12345040,900

11-JUN-2007-05-00,12345010,375

11-JUN-2007-05-00,12345020,900

11-JUN-2007-05-00,12345030,825

11-JUN-2007-05-00,12345040,450

11-JUN-2007-05-00,12345050,1125

11-JUN-2007-06-00,12345010,1425

11-JUN-2007-06-00,12345020,900

11-JUN-2007-06-00,12345030,1425

11-JUN-2007-07-00,12345010,1050

11-JUN-2007-07-00,12345020,225

11-JUN-2007-07-00,12345030,525

11-JUN-2007-07-00,12345040,525

11-JUN-2007-07-00,12345050,750

11-JUN-2007-07-00,12345060,375

11-JUN-2007-08-00,12345010,1350

11-JUN-2007-08-00,12345020,1275

11-JUN-2007-08-00,12345030,1125

11-JUN-2007-09-00,12345010,825

11-JUN-2007-09-00,12345020,75

11-JUN-2007-09-00,12345030,300

11-JUN-2007-09-00,12345040,1500

11-JUN-2007-09-00,12345050,75

11-JUN-2007-09-00,12345060,1200

11-JUN-2007-10-00,12345010,600

11-JUN-2007-10-00,12345020,450

11-JUN-2007-10-00,12345030,150

11-JUN-2007-10-00,12345040,1425

11-JUN-2007-10-00,12345050,1200

11-JUN-2007-11-00,12345010,525

11-JUN-2007-11-00,12345020,825

11-JUN-2007-11-00,12345030,1275

11-JUN-2007-11-00,12345040,1200

11-JUN-2007-12-00,12345010,525

11-JUN-2007-12-00,12345020,675

11-JUN-2007-12-00,12345030,1275

11-JUN-2007-12-00,12345040,225

11-JUN-2007-12-00,12345050,675

11-JUN-2007-12-00,12345060,600

11-JUN-2007-13-00,12345010,1425

11-JUN-2007-13-00,12345020,900

11-JUN-2007-13-00,12345030,900

11-JUN-2007-13-00,12345040,375

11-JUN-2007-14-00,12345010,900

11-JUN-2007-14-00,12345020,825

11-JUN-2007-14-00,12345030,450

11-JUN-2007-14-00,12345040,1125

11-JUN-2007-14-00,12345050,1425

11-JUN-2007-15-00,12345010,900

11-JUN-2007-15-00,12345020,1425

11-JUN-2007-15-00,12345030,1050

11-JUN-2007-15-00,12345040,225

11-JUN-2007-16-00,12345010,525

11-JUN-2007-16-00,12345020,525

11-JUN-2007-16-00,12345030,750

11-JUN-2007-16-00,12345040,375

11-JUN-2007-16-00,12345050,1350

11-JUN-2007-17-00,12345010,1275

11-JUN-2007-17-00,12345020,1125

11-JUN-2007-17-00,12345030,825

11-JUN-2007-17-00,12345040,75

11-JUN-2007-17-00,12345050,300

11-JUN-2007-18-00,12345010,1500

11-JUN-2007-18-00,12345020,75

11-JUN-2007-18-00,12345030,1200

11-JUN-2007-18-00,12345040,600

11-JUN-2007-18-00,12345050,450

11-JUN-2007-19-00,12345010,150

11-JUN-2007-19-00,12345020,1425

11-JUN-2007-19-00,12345030,1200

11-JUN-2007-19-00,12345040,525

11-JUN-2007-19-00,12345050,825

11-JUN-2007-20-00,12345010,1275

11-JUN-2007-20-00,12345020,1200

11-JUN-2007-20-00,12345030,525

11-JUN-2007-20-00,12345040,675

11-JUN-2007-21-00,12345010,1275

11-JUN-2007-21-00,12345020,225

11-JUN-2007-21-00,12345030,675

11-JUN-2007-21-00,12345040,600

11-JUN-2007-21-00,12345050,1425

11-JUN-2007-22-00,12345010,900

11-JUN-2007-22-00,12345020,900

11-JUN-2007-22-00,12345030,375

11-JUN-2007-22-00,12345040,900

11-JUN-2007-22-00,12345050,825

11-JUN-2007-23-00,12345010,450

11-JUN-2007-23-00,12345020,1125

11-JUN-2007-23-00,12345030,1425

11-JUN-2007-23-00,12345040,900

12-JUN-2007-14-00,12345010,1425

12-JUN-2007-14-00,12345020,1050

12-JUN-2007-14-00,12345030,225

12-JUN-2007-14-00,12345040,525

12-JUN-2007-14-00,12345050,525

13-JUN-2007-15-00,12345010,750

13-JUN-2007-15-00,12345020,375

13-JUN-2007-15-00,12345030,1350

13-JUN-2007-15-00,12345040,1275

14-JUN-2007-16-00,12345010,1125

14-JUN-2007-16-00,12345020,825

14-JUN-2007-16-00,12345030,75

14-JUN-2007-16-00,12345040,300

14-JUN-2007-16-00,12345050,1500

15-JUN-2007-14-00,12345010,75

15-JUN-2007-14-00,12345020,1200

15-JUN-2007-14-00,12345030,600

15-JUN-2007-14-00,12345040,450

15-JUN-2007-14-00,12345050,150

15-JUN-2007-14-00,12345060,1425

16-JUN-2007-15-00,12345010,1200

16-JUN-2007-15-00,12345020,525

16-JUN-2007-15-00,12345030,825

16-JUN-2007-15-00,12345040,1275

17-JUN-2007-16-00,12345010,1200

17-JUN-2007-16-00,12345020,525

17-JUN-2007-16-00,12345030,675

17-JUN-2007-16-00,12345040,1275

18-JUN-2007-14-00,12345010,225

18-JUN-2007-14-00,12345020,675

18-JUN-2007-14-00,12345030,600

18-JUN-2007-14-00,12345040,1425

18-JUN-2007-14-00,12345050,900

19-JUN-2007-15-00,12345010,900

19-JUN-2007-15-00,12345020,375

19-JUN-2007-15-00,12345030,900

19-JUN-2007-15-00,12345040,825

19-JUN-2007-15-00,12345050,450

19-JUN-2007-15-00,12345060,1125

20-JUN-2007-16-00,12345010,1425

20-JUN-2007-16-00,12345020,900

20-JUN-2007-16-00,12345030,1425

21-JUN-2007-14-00,12345010,1050

21-JUN-2007-14-00,12345020,225

21-JUN-2007-14-00,12345030,525

21-JUN-2007-14-00,12345040,525

21-JUN-2007-14-00,12345050,750

21-JUN-2007-14-00,12345060,375

22-JUN-2007-15-00,12345010,1350

22-JUN-2007-15-00,12345020,1275

22-JUN-2007-15-00,12345030,1125

23-JUN-2007-16-00,12345010,825

23-JUN-2007-16-00,12345020,75

23-JUN-2007-16-00,12345030,300

23-JUN-2007-16-00,12345040,1500

23-JUN-2007-16-00,12345050,75

23-JUN-2007-16-00,12345060,1200

24-JUN-2007-14-00,12345010,600

24-JUN-2007-14-00,12345020,450

24-JUN-2007-14-00,12345030,150

24-JUN-2007-14-00,12345040,1425

24-JUN-2007-14-00,12345050,1200

25-JUN-2007-15-00,12345010,525

25-JUN-2007-15-00,12345020,825

25-JUN-2007-15-00,12345030,1275

25-JUN-2007-15-00,12345040,1200

26-JUN-2007-16-00,12345010,525

26-JUN-2007-16-00,12345020,675

26-JUN-2007-16-00,12345030,1275

26-JUN-2007-16-00,12345040,225

27-JUN-2007-14-00,12345010,675

27-JUN-2007-14-00,12345020,600

27-JUN-2007-14-00,12345030,1425

28-JUN-2007-15-00,12345010,900

28-JUN-2007-15-00,12345020,900

28-JUN-2007-15-00,12345030,375

28-JUN-2007-15-00,12345040,900

29-JUN-2007-16-00,12345010,825

29-JUN-2007-16-00,12345020,450

29-JUN-2007-16-00,12345030,1125

29-JUN-2007-16-00,12345040,1425

30-JUN-2007-14-00,12345010,900

30-JUN-2007-14-00,12345020,1425

30-JUN-2007-14-00,12345030,1050

01-JUL-2007-15-00,12345010,225

01-JUL-2007-15-00,12345020,525

01-JUL-2007-15-00,12345030,525

01-JUL-2007-15-00,12345040,750

01-JUL-2007-15-00,12345050,375

01-JUL-2007-15-00,12345060,1350

02-JUL-2007-16-00,12345010,1275

02-JUL-2007-16-00,12345020,1125

02-JUL-2007-16-00,12345030,825

03-JUL-2007-14-00,12345010,75

03-JUL-2007-14-00,12345020,300

03-JUL-2007-14-00,12345030,1500

03-JUL-2007-14-00,12345040,75

03-JUL-2007-14-00,12345050,1200

04-JUL-2007-15-00,12345010,600

04-JUL-2007-15-00,12345020,450

04-JUL-2007-15-00,12345030,150

04-JUL-2007-15-00,12345040,1425

04-JUL-2007-15-00,12345050,1200

05-JUL-2007-16-00,12345010,525

05-JUL-2007-16-00,12345020,825

05-JUL-2007-16-00,12345030,1275

05-JUL-2007-16-00,12345040,1200